Notes on the CCS Version 5 BIOS (CC19BIOS)

CC19BIOS.ASM has several significant changes in it.

a).  Drives will be deselected when the 2422 signals the heads

to unload.  This causes all drive select lights to go out.

b).  Step rates are easier to change:

8" step rate is stored at BIOS+33H (Default = 6 ms).

5" step rate is stored at BIOS+34H (Default = 30 ms).

c).  Drivers for the CCS 2719 2+2 board are included:

Serial "A" is the default LST: device (LST:=TTY:)

9600 baud, 8 bit, 1 stop bit, no parity

Handshake on RS232 pin 4

(The handshake line may be modified at the port's

configuration header to look at RS232 pin 11, 19

or 20, as desired.  If the printer handshakes on

pin 20, the SIO status routine can be modified to

a mask of 20h instead of 4h.  RS232 pin 4, however,

must be held TRUE by the printer for this to work.)

Serial "B" is the default RDR: (input) and PUN: (output).

9600 baud, 8 bit, 1 stop bit, no parity

Handshake on RS232 pin 20 (output inhibit)

Parallel "1" is the LST:=LPT: device.  Use STAT command

to set this selection.

Parallel "2" is the LST:=UL1: device.  Use STAT command

to set this selection.

d).  Better rewponse the double-sided drives (Qume DT8s especially)

has been built in.

e).  If "DRIVE x NOT READY" occurs, Control C will allow recovery.

Exception:  If one attempts to select a non-existant drive

as the default drive, the only recourse is to re-boot.

f).  Sector skewing algorithm has been simplified.  Skew tables

now only require two entries:  The physical sectors per

track; and the skew factor.

g).  Conditional assembly parameters allow selection of different

5" drive characteristics.  (Single vs. double-sided;

35 track, 40 track, or 80 track.)

CC19BIOS may be installed on either a single density or a double density

diskette.  For single density, prepare a standard single density

system diskette.  Then, prepare a new RLOCBIOS.COM file by first

insuring that the new RLOCBIOS.COM, the new double density bios

(CC19BIOS in this example), SUBMIT.COM, XSUB.COM, ED.COM, ASM.COM,

PIP.COM, DDT.COM, GENMOD.COM, and ASMRLOC.SUB are present on the

A: drive.  Then, type in the following command line:

A:SUBMIT ASMRLOC CC19BIOS

After a little while, a new RLOCBIOS.COM will be present

on the A: drive.  Transfer it onto the single density system diskette

and you are on your way.

Preparation of a double density system diskette still requires

the procedure of Application Note 5 in the CCS Controller Unique Software

manual.

New versions of CCSYSGEN, CCCOPY, and CCSINIT have also been created.

These routines improve operator input response, are easier to modify

the step rates, and in the case of CCSYSGEN, implement the undocumented

capability of DRC's SYSGEN to read in a CPMxx.COM file, if the

desired file is specified on the command line.

To allow the tailoring of specific system step rates, the

program SETSTEP.COM was created.  To use SETSTEP, merely type in:

A>SETSTEP (filename)

and answer the questions.

NOTE: SETSTEP only works on the new CCSYSGEN, CCSINIT, and CCCOPY.

It does not work on CC19BIOS.

NOTE:  Old copies of CCSYSGEN, CCSINIT, and CCCOPY will not return

control to CP/M properly when CC19BIOS is installed.  Changing the

location of the side select bits storage (from 0043H to 0041H)

causes this problem.

Ben Mason   6 April 82

##################