##APPENDIX B ##

##DESCRIPTIO� KEYBOARD,VIDEO DRIVER##

#Channel Assignments# :

KVDDEV - 01 -      Keyboard,video device

#Operation Codes# :

---------------------------------

GSTATRG- 01 -      Get status register

GSTATUS- 02 -      Get status

SSTATUS- 03 -      Set status

READ   - 04 -      Read

WRITE  - 05 -      Write

RDFLUSH- 06 -      Flush input channel

WRFLUSH- 07 -      Flush output channel

#Error Codes# :

NOERR  - 00 - No error

NOTRDY - 05 - Not Ready

OUTPAP - 08 - Out of Paper

FRAMERR- 10 - Frame Error

PARERR - 11 - Parity Error

STRTNF - 13 - Start Not Found

ENDNF  - 14 - End Not Found

I� non-zer� erro� cod� i� returned,th� carr� a�d overflo� bit�  �

will be set and the zero flag will be cleared also.

.pa�

#Functional description of the driver.

GET STATUS REGISTER#

#Input Parameters# :

DL = GIOST

#Output Parameters# :

AX : Status word:

Layout:

#Symbol    Bit       Function#

RXRDY     0         hardware receiver buffer has data

RXBRDY    1         FIFO contains data

TXRDY     2         hardware transmitter buffer is ready for data

DCD       3         DCD line active

TXBRDY    4         FIFO ready for data

CTS       5         CTS active

BREAKC    7         Break received

#Description#

Returns status of input and output channel hardware as well as

input and output FIFO status.

#GET INPUT CHANNEL STATUS#

#Description#

Returns Status of input channel FIFO :

Not ready error (AH=NOTRDY) will be returned if FIFO is empty.

.pa�       #SET PARAMETER#

#Introduction#

This function provides subfunctions to set different operating modes

or to pass parameters to the driver.

#Input Parameters# :

DL:  Function code

CODE IN DL          OPERATION Performed

WRATTB              writes AH to internal attribute byte.

Input:

AH : Attribute byte

Bit       function

0         Enable bit 7

i� thi� bi� i� reset,a� an� wit� 7FȠ wil� b� �

performe� befor� an� characte� wil� b� pu� i� th� FIFO

1�        Wai� i� no� read� els� immediat� retur� (Characte� mode)

2         Reserved

3         Reserved

4         Reserved

5         Reserved

6         String mode

7         Block/string mode enable

DISOUTP             disables output channel.

Input:

AL : 0 for disable , 1 for enable of transmitter

SETOWT              puts ES:DI into the internal address register

of the output channel WAIT routine

Input:

ES:DI  pointing to user  routine

SETOTSK             puts ES:DI into the internal address register

of the output channel START routine

Input:

ES:DI  pointing to user  routine

SETIWT              puts ES:DI into the internal address register

of the input channel WAIT routine

Input:

ES:DI  pointing to user  routine

SETITSK             puts ES:DI into the internal address register

of the input channel START routine

Input:

ES:DI  pointing to user  routine

.pa�     SETIPRC             puts ES:DI into the internal address register

of the input channel subhandler routine

Input:

ES:DI  pointing to user routine

SVHADDR             puts ES:DI into the internal address register

of the Video handler calling routine

Input:

AL=1

ES:DI  pointing to user routine

AL=0      resets video handler to internal VT52 mode.

Description:

Thi� function'� purpos� i� t� replac� th� loca� VT5� �

video handler by an user specific video handler.

If AL=0, the video handler will be reset to VT52 mode.

In this case,this function performs also an undo of the

Cursor disable function (CURSEN with AL=1) and of the

SCRADDR function.CURSEN and SCRADDR are described below.

Please refer also to the description of the VIDEOROUTINE.

CURSE�              enable� o� disable� th� curso� i� dependenc� o� AL.

Input:

AL=0      to disable cursor

AL=1      to enable cursor

AL=2      to put ES:DI in the address register of an

user cursor complement routine.

ES:DI     pointing to user cursor complement routine.

Description:

This function anebles or disable the cursor or replaces

th� interna�curso� complemen�routin� b�a�use� routine.

Pleas� refe� als� t� th�descriptio�o�th� CURSORCPLROUTINE

SCRADDҠ            put� ES:Dɠ i� th� addres� registe� o�

the scroll handler.

Input:

ES:DI     pointing to user scroll routine.

Output:

ES:DI     pointing to scroll request routine.

Description:

Thi� routin� replace� th� interna� scrol� routin� b� a�

user specific scroll routine.

Pleas�refe�also t� th�descriptio� o� th� SCROLLROUTINE.

.pa�       #READ#

#Input Parameters# :

AH : Options

Bit       Function

1�        Wai� i� no� read� els� immediat� retur� (Characte� mode)

2         Reserved

3         Reserved

4         Reserved

5         Enables the internal attribute byte

6         String mode

7         Block/string mode enable

CX : Number of characters in buffer for Blockmode

Start,Stop Character for Stringmode

DX : Buffer size for stringmode (maximum string length)

ES:DI: Buffer pointer

#Output Parameters# :

AL : Character if character mode

Buffer [ES:DI] : Contains received data if Block or string mode

CX : contains length of string received  if in string mode.

#Description#

This function is the basic read function.

WAIT bit:

I� th� wai� bi� i� rese� an� th� outpu� FIFO i� full, th� drive� wil�

exit with a "not ready" error.

If the wait bit is set the driver will call an user programmable routine

which may be an task switching utility.If the driver becomes ready again it

will call an seperate routine which may wake up the task again.

The wait bit is ignored in block and string mode and the driver will always

call an user programmable routine.

The driver provides internaly an dummy routine which will be used until

the user sets the address of an user specific routine.

The driver provides three different operating modes:

Characte� mod� provide� singl� characte� outpu� i� th� string/bloc�

mode bits are reset.

If the block mode bit is set the driver will transfer in fixed number

of characters.

If the block and string mode bits are set,the driver will scan the user

buffe� fo� a� characte� containe� i� CH i� CH � � an� wil�transfer �

unti� a� sto� characte� containe� i� CL wil� b� foun� .

Example:

Start character: ':'

Stop character: '.'

Data received: "12345:DATA.xyz"

Buffer contains: "DATA"

.pa�       #WRITE#

#Input Parameters# :

AH : Options

bit       Function

1�        Wai� i� no� read� els� immediat� retur� (Characte� mode)

2         Reserved

3         Reserved

4         Reserved

5         Enables the internal attribute byte

6         String mode

7         Block/string mode enable

AL : Character if in character mode

CX : Number of characters in buffer for Blockmode

Start,Stop Character for Stringmode

DX : Buffer size for stringmode (maximum string length)

Buffer [ES:DI] : Data to be send if block or string mode.

#Output Parameters# :

#Description#

This is the basic output driver.

WAIT bit:

I� th� wai� bi� i� rese� an� th� outpu� FIFO i� full,th� drive� wil�

exit with an notready error.

If the wait bit is set the driver will call an user programmable routine

which may be an task switching utility.If the driver becomes ready again it

will call an seperate routine which may wake up the task again.

The wait bit is ignored in block and string mode and the driver will always

call an user programmable routine.

The driver provides internaly an dummy routine which will be used until

the user sets the address of an user specific routine.

The driver provides three different operating modes:

Characte� mod� provide� singl� characte� outpu� i� th� string/bloc�

mode bits are reset

If the block mode bit is set the driver will transfer in fixed number

of characters.

If the block and string mode bits are set,the driver will scan the user

buffe� fo� a� characte� containe� i� CH i� CH� � an� wil�transfer �

unti� a� sto� characte� containe� i� CL wil� b� foun�.

Example:

Start character: '!'

Stop character: '#'

User buffer contains: "ABCD!123#EFG"

Data send: "123"

.pa�       #FLUSH INPUT BUFFER#

#Input Parameters# :

AH : Function code to permit copying of data to buffer after flush.

AL : Character to be stored if enabled by AH = STOAL

Buffe� [ES:DIݠ �� Contain� strin� o� #C� byte� lengt� whic� wil�b� �

copyied to buffer after flushing if AH = STOBUFF.

CX : Length of buffer if AH = STOBUFF

#Description#

This function performs an flush of the input buffer unconditionally if

AȠ �� NOOPƠ .Otherwis� i� A� � STOA� the� A� wil� b� stor� a� th� firs� �

character after the buffer has been flushed.

I� AȠ � STOBUF� th� buffe� [ES:DI� o� lengt� C� wil� b� store� i� inpu� �

buffer.

Abov� functio� i� recomende� t� b� use� fo� implementatio� o� a� brea� �

function etc.

#FLUSH OUTPUT BUFFER#

#Description#

This function performs an flush of the output buffer unconditionally.

.pa�               ##Description VT52 functions##

Th� VT5�� emulato� perform� th� followin� functions�� whic� wil� b� �

initiated� b� a� escap� characte� (1� hex�� followe� b� on� ASCI� �

characte� an� optiona� paramete� characters.

#Command#   #Function#

ESC A        Move cursor up one line, stop at top.

ESC B        Move cursor down one line, stop at bottom.

ESC C        Move cursor right one position, stop at right margin.

ESC D        Move cursor left one position, stop at left margin.

ESC H        Move cursor to upper left corner (home).

ESC J        Erase from cursor to end of screen.

ESC K        Erase from cursor to end of line.

ES� � � �    Addres� cursor�� 'y� i� th� vertica� address�� 'x� i� th� �

horizontal address. The cursor address is represented by

row+20He� an� column+20Hex�� I.E�� hom� positio� wil� b� �

addressed by 20H,20H or lower right margin by 38H,5FH.

ESC p        Inverse video on.

ESC q        Inverse video off

Furthermore the following ASCII control codes are supported:

^G           Bell

^H           Back space with or without wraparound in dependence of

setup��� Pleas� refe� t� sectio� ?�� fo� informatio�

regarding setup.

^I           Tabulator mod 8.

^J           Linefeed, scroll up if at bottom.

^M           Return to beginning of current line.

Othe� control� code� hav� n� effec� an� th� characte� se� i� standard�

USASCII.

.pa�          #Interfacing to bit map#

Th� vide� memor� contain� a� imag� o� th� scree� includin� porche� an� �

syn� signals�� Th� imag� o� a� no� interlace� scree� i� divide� int� � �

vertica� partition� (rows)�� an� th� imag� o� a� interlace� scree� i� �

divide� int� � vertica� partition� (rows)�� Eac� ro� i� divide� int� � �

horizonta� partition� (columns).Th� siz� o� eac� vertica� partitio� i� �

define� i� line� eac� lin� consist� o� horizonta� partition� (X1-X5)� �

an� th� siz� o� eac� horizonta� partitio� i� define� i� word� (1� bits)�

Dat� wil� b� store� i� row2,colum� 2� T� perfor� vertica� scrolling� th� �

pointe� i� colum� � o� row� Y� an� Y� (an� Y� i� interlaced�� mus� b� �

modified� Al� othe� row� an� column� can be ignored .

X1      X2      X3      X4     X5

Beginning > pointed to by video base segment)

Y3          | VBAP  | VBAP  | VBAP  | HSYNC | JMP |

Y4          | VSYNC | VSYNC | VSYNC | HSYNC | JMP |

Y�          � VFRO� � VFRO� � VFRO� � HSYNC | JMP |

Y5          | VBAP  | VBAP  | VBAP  | HSYNC | JMP |

Y6          | VSYNC | VSYNC | VSYNC | HSYNC | JMP |

Y7          � VFRO� � VFRO� � VFRO� � HSYNC | JMP |

Y�          � HFRO� � FIEL� � HBA�  � HSYNC | JMP | < End of screen

Y2a         | HFROP | FIELD | HBAP  | HSYNC | JMP | < End of memory

Row Y1 relates to vertical front porch.

Row Y2 relates to active field.

Row Y2a relates to dummy lines.

Row Y3 relates to vertical back porch.

Row Y4 relates to vertical sync.

Rows 5 to 7 are only existing if interlaced video.

Row Y5 relates to vertical front porch of frame 2.

Row Y6 relates to vertical back porch of frame 2.

Row Y7 relates to vertical sync of frame 2 .

Col X1 relates to horizontal front porch.

Col X2 relates to active field.

Col X3 relates to horizontal back porch.

Col X4 relates to horizontal sync.

#Accessing the video memory#

Al� relevan� parameter� ar� store� i� a� ra� base� paramete� table.Th� �

addres� o� thi� tabl� i� store� i� GIFDIC'� paramete� table.Th� entr� i� �

VIDPART�� Thi� entr� represent� th� offse� fro� th�  pro� dat� segmen� �

(SDGROUP�� t� th� beginnin� o� th� vide� paramete� table.T� acces� th� �

vide� paramete� table�� i� i� recommende� tha� th� designe� make� usag� �

o� th� fil� VIDS.S86�� whic� contain� � structur� neccessar� t� handl� �

th� dat� area.�.pa     �

#Description video parameter table#

Size  Structure     Description

Entry

�������������������

Byte  FLAGS����flag byte.

Byte  LCHAR����lines rows per character.

Word  LINES����tota젠 numbe� o� line� includin� porche�  an� syn� �

���������������fields (Y1-7).

Word� WORDS ���th�  numbe�  o�  word� pe� line�� includin�  porche� an� �

���������������syn� fields (X1-5).

Word  ALINES ��number of active lines (Y2).

Word  AWORDS  �active words per line (X2).

Word� VSEG� ���vide� bas� segment�� al� offset� i� thi� tabl� ar� �

���������������reference� t� thi� segment.

Word  VSYNCF1� offse� t� vertica� syn� fiel� 1�� Thi� paramete� i� �

���������������require� fo� scrolling.

Word  VSYNCF2��same as above, but for frame 2 if interlaced video.

Word  VSYNCE1��offse� t� exi� addres� (jm� pointer� ver� syn� fiel� 1� �

���������������Required for scrolling.

Word  VSYNCE2��same as above, but for frame 2 if interlaced video.

Word  FRSTLIN��offse� t� dat� fiel� o� firs� activ� line�� Thi� pointe� �

���������������i� require� i� conjunctio� wit� FHB� t� acces� th� scree� �

���������������memory.

Word  OFFSEX���offse� fro� beginnin� o� lin� t� exi� addres� (jm� �

���������������pointer of line. Required for scrolling.

Word  OAWRDS���tota� numbe� o� word� (size�� o� vide� memory�

Word  DUMMYL���numbe� o� dumm� line� behin� las� activ� line�� Thes� �

���������������lines will be used to buffer multiple character lines for

���������������faster scrolling or for smooth scrolling (Y2a).

Word  UTABLE���characte� generato� bas� segment�� Reqire� t� acces� th� �

���������������loca� characte� generator��� Note��� a� use� specifi� �

���������������characte� generato� ma� b� mappe� i� b� loadin� it� star� �

���������������addres� i� thi� memor� location�� Refe� t� CHARACTE� �

���������������GENERATOR.

Dword IATTR����dwor� pointe� t� attribut� initialisatio� routine�� Refe� �

���������������t� INITIALISATIO� o� video.

Dword SLNKLST��dwor� pointe� t� lin� lis� initialisatio� routine�� refe� �

���������������to INITIALISATION of video.

Word  BYTES����total bytes per line ((X1+X2+X3+X4+X5)*2).

Word  FHBL�����lengt� o� horizonta� fron� porc� � horizonta� bac� porc� �

����������������� horizonta� syn� width�� Permit� pointin� fro� en� o� �

���������������activ� fiel� o� on� lin� t� star� o� activ� fiel� o� nex� �

���������������physical line (X1+X3+X4+X5).

Word  OFFSNXT��offset to next character line. Equivalent to BYTES*LCHAR.

Word  SCRNMAX��Offset to last byte on screen.

Word  SCRNLEN��length of active screen. Equivalent to ALINES*BYTES.

Byte  ABYTES���characters per line. Equivalent to AWORDS*2 (X2*2).

Byte  XMAXI����maximum X cursor (0 origined).

Byte  YMAXI����maximum Y cursor (0 origined).

Byte  YNR������number of reserved character lines.

Byte  YND������number of dummy character lines.

���������������

.pa�          #Initialisation of video#

T� allo� acces� t� th� vide� ram�� i� i� require� t� gai� acces� t� th� �

vide� paramete� tabl� describe� above�� Thi� wil� b� don� b� readin� �

entry VIDPART of the GIFDIC address table. Afterwards, it it recommended

tha� th� lin� lis� wil� b� initialise� t� allo� linea� accesse� t� �

scree� memory�� A� exampl� procedur� is:

INCLUDŠ� GIFMAC.MAC ;INCLUDE EQUATES,MACROS,STRUCTURES

INCLUDE   VIDS.S86

VINIT     PROC      NEAR

MOV       CX,0       ;ENQUIRE ADDRESS OF GIFDIC

INT       99H        ;ADDRESS TABLE (INTO ES:DI)

MOV       PROMDATA,ES ;SAVE PROMDATA SEGMENT

MOV       ADDRTAB,DI ;SAVE OFFSET TO ADDRESS TABLE

MOV       DI,ES:[DI.VIDPART] ;AX=OFFSET TO TABLE

MOV       VIDTAB,DI  ;SAVE FOR FUTURE USE

CALL      DWORD PTR ES:[DI.SLNKLST] ;RESET LINK LIST

RET

VINIT     ENDP                 ;THAT'S ALL

T� initialis� th� vido� attributes�� th� routin� [VIDPART.IATTR� ha� t� �

b� executed�� Thi� wil� b� don� automaticall� a� syste� initialisatio� �

time, and is not required afterwards.

.pa�          #Character generator#

Entr� UTABL� o� th� vide� paramete� tabl� i� displacemen� fro� PGROU� t� �

beginin� o� th� loca� characte� generato� table� Thi� tabl� contain� 12� �

10 Byte entrys to support standard ASCII characters from 20H to 7EH.

.PA�ref 1.1 17.feb 85

#################################