ADDING NEW CRT'S TO VEDIT (8080 and Z80 only)

Please follow this procedure to modify the CRT tables used

in  customizing  VEDIT   for  different   CRT  terminals.  This

procedure allows you to make the changes necessary for VEDIT to

support a terminal which  is  not  listed  in  the CRT terminal

menu. The following  assumes  that  'CRT.ASM'  is  present on a

logged-in  drive  'B',   and  that  the   files  'ASM.COM'  and

'LOAD.COM' are present on drive 'A'.

1.  Select a CRT  entry in  the file  CRT.ASM for modification.

(Usually CUSTOMER 2 for new CRT terminals)

2.  Edit the  relevant  table.  Look  at  the  comments  at the

begining of CRT.ASM  for information about  what each entry

(line) in the table represents.

3.  Give the following commands:

'A:ASM CRT' <RET>

'A:LOAD CRT' <RET>

'ERA CRT.TBL' <RET>    (This purges the old CRT.TBL)

'REN CRT.TBL=CRT.COM' <RET>

4.  Now you may re-customize VEDIT and choose the number of the

CRT terminal which corresponds  to the crt  entry which has

been modified.

NOTES:  1. Note that you may only  modify an existing crt table

in the file CRT.ASM, and that an error will occur if

you attempt to  add  another  entry  to  the CRT.ASM

file.

2. If you find  that you must  debug your modifications

for a new CRT terminal, you  may wish to examine the

'VEDITZC.LST'  or  'VEDIT8C.LST'  file.  This  shows

where the specific  CRT table  entries reside within

the customized  VEDIT. If  you are  experienced with

DDT, these may  be modified while  VEDIT is running.

If you debug VEDIT with  DDT, you may re-enter VEDIT

at location 103 Hex (4303 Hex for ORG 4200 system.)

############################################################