PMMIBYE5.DOC    (as of 7/20/80)

The PMMIBYE5.ASM program is out for distribution,

but it has not been thoroughly tested in all of the

possible modes of operation.

(Please note that version 5.4 is substantially longer

than previous PMMIBYE versions.  Because of the great

number of conditional switches, the program could require

as many as 1060 bytes of RAM, or it could be smaller than

the 900 bytes used by the original PMMIBYE.  It all

depends on the way you set it up.)

Basically, several new conditional assembly routines

have been added to allow user selection at time of

assembly of the following:

1. Password access

2. User log (counts number of system callers)

3. Memory-mapped console I/O

Also added to V5.4:

1. 710 Baud option for PMMI modem

2. Modifications to AUTO-ANSWER routines

3. "/R" option to allow reset of user log counters

Password access was made conditional to allow

use with systems that offer unrestricted access.

The user log was added to allow the system operators

to keep track of the number of people who called the

system.  In addition, it can also count the number of

unsuccessful log-ons (i.e. didn't know password) and

the number of voice calls, and conditionally

display both numbers on the system console when the

program is exited with a control-C from the system

console.  The counters can be initialized with

the "/R" option to all zeroes, or preset by using

DDT, or SID.  The number of successful log-ons is also

conditionally output to port 255 (0FFH) for display on the

computer's front panel.

The memory-mapped I/O conditionals have been added

to allow use of the serial I/O port on the Morrow's 2D

controller board as the system console; however, the

routines can be used with any memory-mapped system.

The 710 Baud option was added for "short-haul"

callers, and because operation at 710 Baud with

the PMMI modem is "iffy" at best, it will probably

be of very little use.

The AUTO-ANSWER routines were modified to allow

more dependable ringback operation.  Unfortuneately,

the ringing signal at the caller's end of the phone line

does not always correspond exactly with the ringing

signal at the system's end.  Although a system-end ring

does occur sometime during the caller's ring cycle (ring-

no ring), it is possible for the system to receive no

or 2 rings in the time that the caller thinks just

one ring has occurred (The DIAL program is notorious for

this).  The solution chosen for this problem was to

check for one or two rings before call-back.  This

would allow for the "phase" difference of the caller-

system ring signals.  This solution also means that

the phone must ring at least three times before the

system will assume that the call is voice, and is not

to be answered.  The advantage of this arrangement is

that it is virtually impossible for the computer to

"miss" a call that is intended for it.

#ARD/MODEM STATUS TEST ROUTINE

;

MSTAT	EQU	$+OFFSET

;

IF	DUAL$IO