VEDIT - Visual Editor					      Page 52

Appendix A - Customizing with VEDSET

Customizing VEDIT

Introduction

------------

VEDIT has to be customized before the  first time it is used, and

can then be customized again, when the  user has a new keyboard, a new

CRT terminal, just wishes  to change some default  parameters or try a

new keyboard  layout.  Please  note  that  it  does  not  have  to  be

customized every time  you use  it.  The greatest  benefit you receive

from the customization  process is  probably the  ability to determine

your own keyboard layout, which can  utilize any special function keys

and accommodate personal preferences.

VEDIT is supplied  as a  disk file  with an  extension of ".SET",

i.e. VEDITZM.SET, VEDITZC.SET,  which contains  the "prototype" editor

to be customized.  The customization  process does not  alter the .SET

file, but rather creates a new file  with the file extension of ".COM"

which is the executable version of the customized editor.

The customization is  done with the  supplied programs VEDSET.COM

for the memory mapped versions, and  VDSETCRT.COM for the CRT terminal

versions.  Running VEDSET  (or  VDSETCRT)  simply  involves  typing  a

control key or a  number in response to  the questions it asks.  Since

the customization program is fairly easy to run, you will probably run

it several times  in the  first week  until you  have everything "just

right".  You can of course also create several versions of VEDIT, each

for a special application.

Getting started

---------------

The following paragraphs  describe  the  various  aspects  of the

customization in  some detail.  You  do not  need to  fully understand

these details in order to get VEDIT up and running, since the enclosed

"Example Keyboard Layout"  and the  next section  "Running VEDSET ..."

give  recommended  values  for  every  question.  Once  you  are  more

familiar  with  VEDIT,  you  will  probably  want  to  gain  a  better

understanding  of  the  customization  in   order  to  create  a  more

"personalized" version of VEDIT.

Determining the desired  keyboard layout for  the cursor movement

and function keys is  the first  step of  the customization.  Since it

could be  a  difficult  step,  several  example  keyboard  layouts are

enclosed to help  out the  new user.  The  best layout  will depend to

some extent upon your keyboard, especially  if you have one with extra

keys which produce  control codes.  If  extra keys  are available, you

may want to allocate them  to the most used  visual operations such as

the cursor movements.  The  more extra  keys you  have, the  easier it

becomes to remember the layout.

VEDIT - Visual Editor					      Page 53

Appendix A - Customizing with VEDSET

If and when you decide to try  out your own layout, you will want

to avoid placing the keys  you least want to  hit by accident, such as

[Erase End Of Line] or [Home], right next to the cursor movement keys.

In the event  that  you  have  no  or  few  special  keys, most visual

operations will involve  holding  the  CONTROL  key  while  you type a

letter, or using escape  sequences.  In this  case, the  layout may be

tight and difficult  to  organize.  One  strategy  is  to use mnemonic

letters, such as CTRL-D for [DELETE]  and CTRL-U for [CURSOR UP], etc.

Another is to arrange  the keys  in some  logical manner,  such as the

cursor movement keys  on  one  side  of  the  keyboard  and the visual

function keys on the other side.  You  can also simplify the layout by

using at least a few escape sequences, especially for functions you do

not use  often, or  don't want  to hit  by accident.  Trying  out some

combinations on paper is  probably the  easiest way  to accomplish the

layout task.

Besides responding to  the  customary  control  characters, VEDIT

also handles multi  character  escape  sequences.  These  may  be user

typed, or  may  result  from  pressing  a  special  function key.  For

example, instead of  typing the  single character  CONTROL-Q, the user

may type  two  characters,  i.e.  ESC  and  Q,  to  perform  a  visual

operation.  All escape sequences  begin with  one of  two user defined

escape characters (sometimes  called  Lead-in  characters).  While the

ESC is a common  key to  use as  an escape  character, any other ASCII

character may be used  as the escape  character, even displayable ones

like "@".  The special function keys on some keyboards, like the Heath

H19, Televideo 920C and  IBM  3101  also  send  multi character escape

sequences.  Some terminals, like  the IBM  3101, also  send a Carriage

Return at the  end  of  escape  sequences.  The keyboard customization

detects this automatically and the user need not be concerned with it.

When  laying  out  the  keyboard,   you  may  therefore  use  any

combination of control  characters, special  function keys  and escape

sequences for the  visual operations.  Some  users will  prefer to use

function  keys  and  control  characters  for  the  most  used  visual

operations, and escape  sequences  for  the  less used operations.  If

escape sequences are used,  a key like  ESC or FORM  FEED is suggested

for the escape mode  character.  Any other character  may then follow,

including  numbers,   control  characters   or  even   another  escape

character.  Many keyboards have a numeric pad and these numbers can be

used in escape sequences.  For  example, use ESC -  8 for [CURSOR UP],

ESC - 2 for [CURSOR  DOWN], ESC -  4 for [CURSOR  LEFT] and so on.  In

this case you  may wish  to attach  descriptive lables  on top  of the

numeric keys.  An Escape and Control  character combination would be a

good choice  for operations  you don't  want to  hit by  mistake, like

[HOME], [ZEND] or  [RESTART EDITOR].  You  may use  an escape sequence

consisting of two escape characters in a  row.  In fact, if ESC is the

escape character, then "ESC - ESC"  is the suggested  sequence for the

function [VISUAL ESCAPE].  In  the  unusual  case  that  a displayable

character like "@" is  used as the escape  character, a "@ - @" cannot

be used for a  visual operation, since in  this case,  "@ - @" will be

treated by VEDIT as the normal "@" character.

VEDIT - Visual Editor					      Page 54

Appendix A - Customizing with VEDSET

While all of this is complicated  enough already, there are a few

pitfalls to avoid  too.  (You  are  well  advised  to  use  one of the

example keyboard layouts at  first.) The only  key which is predefined

is the RETURN or CR  key which is  also CTRL-M and  cannot be used for

any visual operation.  The  special  function  keys  on some keyboards

send a code  identical  to  a  control  character.  You  may therefore

unintentionally attempt to  use the  same control code  for two visual

operations.  In this case,  VEDSET  or  VDSETCRT  will  give  an error

message and request a new key  for that function.  Some keyboards have

special function keys which  send  a  character  with  data  bit 7 set

(sometimes called the  parity  bit).  These  work  properly  since the

VEDIT programs decode all 8 bits.  (Technical note: An escape sequence

treats the second character  as  having  Bit  7  set.  The escape mode

characters themselves must not have Bit 7 set.)

The second decision  during  customization  is  to  determine the

desired Tab positions and whether tabs should be expanded with spaces.

Unless you have  some  special  application,  don't  expand  tabs with

spaces, it will  use up  lots of  disk space.  Where  you set  the tab

positions will only  be applicable  to VEDIT  since most  CP/M utility

programs set the tab  positions at  every 8th  position.  This is thus

the best choice for VEDIT, too.  One exception would be where you do a

lot of word processing with the same tab positions.  Another exception

would be if you are using a structured language compiler which perhaps

set tabs at every  4th position for  easier indenting.  The values you

enter for the tab  positions, the  switches and  the parameters below,

are the defaults, they  may be changed  while running VEDIT.  Assuming

you want the tabs at every 8th position, the tab positions would be:

9 17 25 33 41 49 57 65 73 81 89 97 105 113 121 129 etc.

These are also the default positions.

Five special characters can also be customized.  The first is the

line continuation indicator used in visual  mode in reserved column 0.

We suggest a "-"  or reverse video  "-", codes "2D"  or "AD" hex.  The

second is the command mode  "Escape character".  This will normally be

the "ESCAPE" or "ESC" key  with a hex code  of "1B".  If your keyboard

does not have an "ESC" key, you will need to choose some other control

character, perhaps a "CTRL-Z"  with a hex code  of "1A".  The next two

characters are the enclosing brackets  for iteration macros.  They are

printed as "[" and  "]" in  this manual  (codes 5B  and 5D hex).  Some

users may be more familiar with the  " " and " " angle brackets (codes

3C and 3E  hex).  Use either  set, but  it may  help if  your keyboard

produces one set without  needing the Shift  key.  The fifth character

only applies to memory mapped versions,  and is the character used for

the blinking "underline"  cursor.  While  this  would  normally be the

underline character, (code 5F  hex), users with  displays which do not

produce reverse video, such  as the  Sorcerer, may  wish to  try a hex

code of "7F" which is commonly a solid block.

VEDIT - Visual Editor					      Page 55

Appendix A - Customizing with VEDSET

You even have  the choice of  whether the messages  on the status

line appear in reverse  video  on  CRTs  which  support reverse video.

Some displays, such  as on  the Sorcerer  and TRS-80  Model I,  do not

produce reverse video.

Next you have to  decide on the default  settings of several more

switches and parameters (switches (2) - (7) and parameters (1) - (5)).

Remember that the switch  and parameter settings  can be changed while

running  VEDIT.  Switch  (2)  determines   whether  auto-buffering  is

enabled during  visual mode.  The  first time  around, we  suggest you

enable auto-buffering.  After some practice and reading the section on

Auto Read/Write,  you  may  decide  otherwise.  Switch  (3) determines

whether VEDIT starts in  Visual mode or  command mode.  The first time

around, we suggest  you  set  this  switch  on.  Switch (4) determines

whether the edit pointer's  position (or  cursor's in  Visual mode) is

moved just past  the newly  inserted text,  when the  text register is

inserted.  Again, for the first  time around, we  suggest that you set

this switch on.  After some practice with  the text register, you will

know which way you prefer to  have this switch.  Switch (5) determines

whether the difference between upper and lower case letters is ignored

in  searches.  We  suggest   you  set  this   switch  on.  Switch  (6)

determines whether the screen is cleared when visual mode is exited to

command  mode.  We  suggest  you  set  this  switch  off.  Switch  (7)

determines whether all letters typed on  the keyboard will be reversed

with regard  to upper  and lower  case, i.e.,  upper case  letters are

converted  to  lower  case  and  vise  versa.  Only  in  very  unusual

situations would you want to  set this switch on,  so set it off.  For

the TRS-80 Model I, you should set  this switch on, since the keyboard

reverses upper and lower case.

Parameter (1) is only  applicable to memory  mapped versions, and

determines  the   cursor  type.  The   cursor  types   are  0=Blinking

Underline, 1=Blinking  Reverse  Video  Block,  2=Solid  Reverse  Video

Block.  Most users seem to  prefer type "1",  but you must  use "0" if

your display does not produce reverse video.  Parameter (2) determines

the memory mapped cursor's blink rate.  Start with the value suggested

by the   VEDSET   prompt.  Parameter   (3)   determines   the  "Indent

Increment".  A value  of  4  is  common  when  structured  programming

languages are being used.  Parameter (4)  controls lower to upper case

conversion.  This is described  under the  "ES" command.  Start with a

value of "0" for no conversion.  Parameter (5) is related to parameter

(4) and again described under the  "ES" command.  Supplying a value of

"3B" hex, makes the ";" the special conditional character.

Two more parameters that can be customized are dependent upon the

memory size  of  CP/M  you  are  running.  For  details  on  these two

parameters  please  refer  to  the  section  below.  While  these  two

parameters can be specified for many  special applications, it is best

to follow the table below  the first few times,  until you have a good

'feel' of the  operational characteristics of  VEDIT.  The first value

must be  specified in  bytes between  1024 and  32768, and  the second

value must be specified in K bytes between 1 and 32.  (A "K byte" is a

VEDIT - Visual Editor					      Page 56

Appendix A - Customizing with VEDSET

unit of 1024 bytes. 1024 = 2 ** 10.)

CP/M size	Value for Spare		Value for Transfer

16K		1526			2

20K		2304			3

24K		3072			4

28K		4096			5

32K		4096			6

36K		5120			7

40K		6144			8

44K		6144			9

48K		7168			10

52K		7168			11

56K		8192			12

60K		8192			13

64K		8192			14

(In particular, do not make the "Spare Memory for Auto-Read" more than

2 times  larger than  the value  in the  table, or  you may  produce a

non-operational editor.  This value is NOT  the amount of memory VEDIT

will use for  the text  buffers, since  VEDIT always  sizes memory and

uses all that is available.  Rather, this value is the number of bytes

that is free in the  text buffer after a file  is read which is larger

than the available  memory space.  For  example, in  a 56K  system the

available memory is about 41K.  If the table value of "8192" was used,

and a very large file  edited, VEDIT would initially  read in only the

first 33K  of  the  file,  leaving  "8192"  bytes  free.  This  can be

verified with the "U" command.)

The last information  needed  for  customization  pertains to your

display board or CRT terminal. First,  you  need to know the number of

lines and the number of characters per line that it produces.  16 x 64

and 24 x 80 are  the most common values.  You  also have the choice of

how many columns on a line are  actually used.  You want to use all of

them, unless you have a special application or unusual hardware.

For the  memory  mapped  versions,  you  also  need  to  know the

beginning address of the  display board  in memory  in hexadecimal and

whether it requires any data bytes output  to a port to initialize it.

For example, many 16 x 64 boards have an address of CC00 hex.  Most of

these 16 x  64 boards  do not  need any  initialization, one exception

being the  Procesor Technology  VDM  board,  which  should  have  a 00

output to Port C8  hex.  (The SOL-20 requires  a 00 output  to Port FE

hex).

VEDIT - Visual Editor					      Page 57

Appendix A - Customizing with VEDSET

Running VEDSET or VDSETCRT

--------------------------

The customization is  done with the  supplied programs VEDSET.COM

for the memory mapped versions, and  VDSETCRT.COM for the CRT terminal

versions.  Depending upon which  version you  have, your  diskette may

contain several ".SET" files,  which will  be described  on a separate

sheet entitled  "Description of Files".

Assuming you wish to  customize the Z80 CRT  version, with a file

name of VEDITZC.SET, the  customized editor is to  be called VEDIT and

the files VEDITZC.SET and VDSETCRT.COM are  on the currently logged in

disk, the command to run VDSETCRT would be:

VDSETCRT VEDITZC VEDIT

A similar command for the 8080 Memory Mapped version would be:

VEDSET VEDIT8M VEDIT

VEDSET (VDSETCRT)  will  now  prompt  with  questions  which  are

answered by typing the control  keys to setup the  keyboard, or with a

number, or 'Y' for  Yes or 'N'  for No.  The questions  with a numeric

answer also require a RETURN at the end of the line.  Unless otherwise

specified, typing a  RUBOUT or CTRL-U  will ignore the  input for that

question and repeat  the last question.  The  following steps describe

the answer to each question.

1.)   (CRT Terminal Only)

The CRT customization program begins by displaying a menu of CRT

terminals which are directly supported.  Type  any key after the

first part  of the  menu, since  the menu  is two  screens long.

When prompted, enter the  number of  the CRT  terminal which you

wish  to  use   VEDIT  on.  You   may  of   course  perform  the

customization  several  times   for  different   CRT  terminals.

Reference the disk file README.CRT in  case your CRT terminal is

not listed in the menu.

2.)   ENTER ESCAPE MODE CHARACTER #1

If you choose to use escape sequences, or your keyboard produces

escape sequences with  special  function  keys,  type the escape

character, or the function key  lead-in character, most commonly

ESC.  Else type RETURN.

ENTER ESCAPE MODE CHARACTER #2

A second escape mode character  may also be specified, typically

for  other  function  keys.  If  not  needed,  type  RETURN.  (A

"CTRL-A" for the Televideo 920C).

ENTER COMMON 2ND CHARACTER IN ESCAPE SEQUENCE

Simply answer with  a  [RETURN]  if  you  are  not  using escape

VEDIT - Visual Editor					      Page 58

Appendix A - Customizing with VEDSET

sequences  or  are  typing  them   in  by  hand.  However,  some

terminal's  special  function  keys   send  3  character  escape

sequences where the  second  character  is  always  the same and

should be ignored.  In this  case type in  the second character.

(A "?" for the Heath H19)

TYPE CONTROL CHARACTERS FOR ....

When prompted for each visual operation, you may press a special

function key, a control  character or enter  an escape sequence.

Disallowed characters  are  the  normal  displayable characters.

Typing one of these  will give an error  and a reprompt.  If you

inadvertently attempt to use  the  same  key  code  for a second

operation, an error  and a  reprompt for  the operation  will be

given.  If you do not  want to  use a  particular function, just

type [RETURN]  to ignore  the function.  Specifically,  you will

probably want to use either [SET  INSERT MODE] and [RESET INSERT

MODE] or [SWITCH INSERT MODE], but not all three functions.  You

probably  won't  use  [RESTART],  since  the  function  is  also

available  in  command  mode.  Otherwise  choose  something  for

[RESTART] which you are very  unlikely to hit by mistake.  Don't

confuse [TAB CURSOR]  with  the  tab  character,  since  it is a

cursor movement operation.  If  you  make  a  mistake, just type

[RETURN] for the  rest  of  the  functions,  since the following

question will let you start over again.

WAS KEYBOARD LAYOUT CUSTOMIZED CORRECTLY? (Y OR N)

Enter 'Y' if the keyboard layout was customized correctly.  Else

enter 'N' to repeat this step.

3.)   DO YOU WISH TO USE THE DEFAULT TAB POSITIONS? (Y OR N)

Enter "Y" if you want  the tabs at every  8th position, which is

the normal for  CP/M.  Otherwise,  enter  "N"  and the following

message appears:

ENTER UP TO 30 TAB POSITIONS IN DECIMAL

Enter the desired  tab  positions,  separating  the numbers with

spaces or commas and  following the  last number  with a RETURN.

Don't be concerned if your input line goes off the right side of

your terminal or screen.  Note that you  need no tab at position

1 and that  the positions  are counted  starting from  1, not 0.

You must also specify at least  one tab position per screen line

and the highest  allowed  position  is  254.  Entering  a number

outside of the range 1-254 will give  an error and a reprompt of

the question.  If you make  a mistake, type  CTRL-U or RUBOUT to

start the question over.

VEDIT - Visual Editor					      Page 59

Appendix A - Customizing with VEDSET

4.)							     Suggest

HEX CODE FOR SCREEN CONTINUATION CHARACTER		2D

HEX CODE FOR COMMAND MODE ESCAPE CHARACTER		1B

HEX CODE FOR COMMAND ITERATION LEFT BRACKET		5B

HEX CODE FOR COMMAND ITERATION RIGHT BRACKET		5D

HEX CODE FOR CURSOR CHARACTER				5F

Enter the number  in  hexadecimal  and  a  RETURN following each

question.  Always enter a value,  there are NO DEFAULTS.  Typing

a CTRL-U or  RUBOUT will  start over  with the  first character.

Note that the last  prompt relating  to the  cursor only appears

with the memory mapped customization.

5.)   (CRT Terminal only)

ENTER DECIMAL VALUE (4MHZ = 76, 2MHZ = 38)

Enter a value of "38"  if you running a  2 Mhz processor or "76"

if you  are running  a 4  Mhz processor.  Interpolate  for other

processor speeds.  This  value  is  only  used  for  CRTs  which

require time delays  for some  functions.  The maximum  value is

255.

6.)   REVERSE VIDEO ON STATUS LINE  (0=NO, 1=YES)

If your CRT  or  video  display  board  produces  reverse video,

answer  with   a  "1"   for  Yes.  If   you  have   a  Sorcerer,

TRS-80 Model I, or some  other  display  which  does not produce

reverse video, answer with a "0" for No.

7.)                                                         Suggest

(1) EXPAND TAB WITH SPACES		(0=NO 1=YES)	0

(2) AUTO BUFFERING IN VISUAL MODE		(0=NO 1=YES)	1

(3) BEGIN IN VISUAL MODE			(0=NO 1=YES)	1

(4) POINT PAST TEXT REG. INSERT		(0=NO 1=YES)	1

(5) IGNORE UC/LC DISTINCTION ...		(0=NO 1=YES)	1

(6) CLEAR SCREEN ON VISUAL EXIT		(0=NO 1=YES)	0

(7) REVERSE UPPER AND LOWER CASE		(0=NO 1=YES)	0

(1) CURSOR TYPE				(0, 1 or 2)	1

(2) CURSOR BLINK RATE (10 is fastest)	(10 - 100)  See Prompt

(3) INDENT INCREMENT			(1 - 20)	4

(4) LOWER CASE CONVERT			(0, 1 or 2)	0

(5) CONDITIONAL CONVERT CHARACTER		(20 - 7E)	3B

Enter the number  in  hexadecimal  and  a  RETURN following each

question.  Type a CTRL-U or RUBOUT to  start over with the first

switch or  parameter.  CRT version  users may  answer parameters

(2) and (3) with an arbitrary  value.  Note that the prompts for

parameters (1) and (2)  relating to the  cursor only appear with

Memory Mapped versions of VEDIT.

VEDIT - Visual Editor					      Page 60

Appendix A - Customizing with VEDSET

8.)   SIZE IN DECIMAL OF SPARE MEMORY FOR AUTO READ

See the table on page 57  for a recommended value depending upon

your memory  size.  Please read  the note  below the  table too.

Enter the decimal number followed  by a RETURN.  The number must

be in the range 1024  - 32768 or an error  and a reprompt of the

question will be given.  Type a CTRL-U  or RUBOUT to restart the

question.

9.)   SIZE IN DECIMAL OF FILE MOVE TRANSFERS IN K BYTES

See the table on  page 57 again  for a recommended value.  Enter

the decimal number  signifying the  multiple of  1K (1024) bytes

desired, followed by a RETURN. The number entered must be in the

range 1 - 32.

10.)  ENTER NUMBER OF SCREEN LINES IN DECIMAL

Enter the  number of  lines on  your CRT  display and  a RETURN.

While most terminals  have 24  lines, some  have a  25th "Status

Line".  On some of these, it is  possible for VEDIT to place its

status line on the 25th line.  These terminals are marked with a

"*" following the terminal's name in  the menu.  To use the 25th

line, answer this question with  a "25".  Note that the Intertec

Intertube II must be specified as having 25 lines.

11.)  ENTER LINE MOVEMENT FOR PAGING IN DECIMAL

Enter  the  number  of  screen  lines  you  wish  [PAGE UP]  and

[PAGE DOWN] to move through the text by.  About 4/5 of the total

number of screen  lines is suggested,  i.e., "12" for  a 16 line

display and "20" for a 24 line display.

12.)  ENTER TOP LINE FOR CURSOR IN DECIMAL

This sets the  top screen  line the  cursor can  normally be on,

before the screen  will begin to  scroll down.  This, therefore,

is the minimum  number of lines  you will always  see before the

line you are editing.  "3" is a good starting point.

13.)  ENTER BOTTOM LINE FOR CURSOR IN DECIMAL

This is similar  to the previous  step, except that  it sets the

bottom line range for  the cursor.  This number  must be greater

than or equal to  the "Top Line  For Cursor" and  at most be one

less than the "Number  of Screen  Lines", since  the very bottom

line is only  used  for  status.  "4"  less  than  the number of

screen lines is a good starting point.

14.)  ENTER SCREEN LINE LENGTH IN DECIMAL

Enter the number of characters per line your CRT display has and

a RETURN.  This number must  be  in  the  range  20 - 255.  This

value will be 80 for most CRT  terminals and either 64 or 80 for

most Memory Mapped  displays.  The  value  for  the MATROX video

display board is 128.

VEDIT - Visual Editor					      Page 61

Appendix A - Customizing with VEDSET

15.)  ENTER LENGTH OF DISPLAYED LINE IN DECIMAL

Enter the number  of  characters  per  line  you  want  VEDIT to

actually display and a  RETURN.  This value will  normally be 80

or 64 since it  usually  is  equal  to  the  screen line length,

unless for some reason  you  don't  wish  to  use  the full line

length.  This number must  be less  than or  equal to  the above

length of a screen line.  The value for the MATROX video display

board is 80.

16.)  (Memory Mapped Only)

ENTER ADDRESS OF SCREEN IN HEXADECIMAL

Enter the memory address of the  beginning of the video board in

hexadecimal and a RETURN.

17.)  (Memory Mapped Only)

ENTER NUMBER OF VIDEO BOARD INITIALIZATION BYTES

Enter "0" if your  board requires no initialization.  Otherwise,

enter a  number between  "1" and  "5", for  the number  of "data

byte" / "port number" pairs needed for initialization.

ENTER [RUBOUT] OR [CTRL-U] TO START PAIR OVER

ENTER DATA BYTE

ENTER PORT NUMBER

The number of 'data byte'/ 'port number' pairs specified must be

entered in hexadecimal  with  each  number  followed  by RETURN.

Typing a CTRL-U or RUBOUT  will ignore  any values for that pair

and reprompt with the "ENTER DATA  BYTE" question for that pair.

(The Processor Technology  VDM board  requires one  pair, a "00"

sent to port "C8" hex,  and the SOL-20 a  "00" sent to port "FE"

hex.)

18.)  WAS THE CUSTOMIZATION DONE CORRECTLY (Y OR N)

Enter 'Y'  if it  appears that  the customization  was performed

correctly, and your customized version  of VEDIT will be created

on disk.  Otherwise, enter 'N'  if you want  to start over again

at step 2.  At step  2  you  can  skip  customizing the keyboard

layout again.

VEDIT - Visual Editor					      Page 62

Appendix A - Customizing with VEDSET

More on the Memory Parameters for Customization.

The first parameter determines how many bytes of memory are free after

VEDIT does an auto-read (such as following an EB command) on files too

large to fit in  memory  all  at  the  same  time.  This  size must be

specified between 1024 and  32768.  A reasonable size  is about 1/4 of

the size of the  text buffer for  small systems and  a little less for

large systems.  The  CP/M operating  system (BDOS  and BIOS)  takes up

about 4K of memory and VEDIT up  to 11K.  The rest of the memory space

is for  the text  buffer and  text register.  Thus  a 24K  CP/M system

would have a 9K buffer, and a  48K system a 33K buffer.  Choosing a 1K

(1024 byte) multiple  makes  the  disk  read/write  work  a little bit

faster.  The second parameter  specifies  the  size  of file transfers

during auto-buffering  and for  the 'N'  command.  See the  section on

auto-buffering for  details.  For normal  use, a  value about  1/3 the

size of the text buffer is  good.  (Specifying a value larger than one

half the maximum text buffer size  may create a non-working version of

VEDIT.)

When auto-buffering is  initiated,  an  attempt  is  made  to read the

number of K bytes  specified during customization  under "Size of File

Transfers".  If there is insufficient memory  space for appending this

many bytes, this  many bytes  are written  from the  beginning  of the

text buffer to the output file.  An  auto-read is then performed which

reads in the rest of the input file,  or until the memory is filled to

within the number of spare  bytes specified during customization under

"Spare Memory for READ".

A Word About Keyboards

With the simplest keyboards,  each visual operation  will have to

be activated  by holding  the CONTROL  key and  typing some  letter or

using an escape sequence.  Moving  up,  keyboards  will  have keys for

Backspace, Tab and  Line  Feed,  which  can  be  used  to  perform the

described function.  Some  keyboards  with  a  numeric  pad  can  send

control codes by holding  the SHIFT or  CONTROL key and  typing one of

the pad keys.  Numeric pad keys  can always be used  as part of escape

sequences.  The  pad  can  then  be  used   for  most  of  the  visual

operations. In some cases, the  keyboard will have many special  keys,

which send a control  code just by  typing one of  them.  In the ideal

case, these control codes will be sent  with the highest data bit set.

(This is Bit 8 and is often called the parity bit.  The ASCII standard

code does not use  Bit 8  and even  a "Full  ASCII" keyboard will send

nothing on Bit  8  or  else  parity  information).  Some  very special

keyboards, usually ones with 70-100 keys on  them, use Bit 8 to decode

all those keys.  Since VEDIT and VEDSET  decode  all 8 data lines from

the keyboard,  these  fancy  keyboards  can  be  used  to  their  full

advantage.

VEDIT - Visual Editor					      Page 63

Appendix A - Customizing with VEDSET

This page is intentionally blank.

VEDIT - Visual Editor					      Page 64

Appendix B - Command reference

'n' denotes a positive number.  'm'  denotes a number which may

be negative to denote backwards in the file.

'string','s1' and 's2'  are  strings  of  characters  which may

include anything except an [ESC]. The special character "|"

will also match any character during the search.

nA		   Append 'n' lines from the input file. (0A)

B		   Move the edit pointer to text beginning.

mC		   Move the edit pointer by 'm' positions.

mD		   Delete 'm' characters from the text.

E		   First letter of extended two letter commands.

nFstring[ESC]	   Search for 'n'th  occurrence of 'string'.

G		   Insert the contents of the text register.

Itext[ESC]	   Insert the 'text' into the text buffer.

mK		   Kill 'm' lines.

mL		   Move the edit pointer by 'm' lines.

nNstring[ESC]	   Search for 'n'th occurrence of 'string' in file.

mP		   Put 'm' lines of text into the text register.

Ss1[ESC]s2[ESC]	   Search for and change 's1' to 's2'.

mT		   Type 'm' lines.

U		   Print # of unused, used and text register bytes.

V		   Go into visual mode.

nW		   Write 'n' lines to the output file. (0W)

Z		   Move edit pointer to end of text.

EA		   Restart the editor. (EX and EB).

EBfile		   Open "file" for Read & Write, perform an auto-read.

EC		   Change disks for reading or write error recovery.

EDfile[ESC]	   Delete (erase) the file "file" from the disk.

EF		   Close the current output file.

EGfile[line range] Insert the specified line  number range of the file

"file" into the text buffer at the edit position.

nEI		   Insert the character whose decimal value is "n".

EP n m		   Change the value of parameter "n" to "m".

1	   Cursor type				(0, 1 or 2)

2	   Cursor blink rate			(10 - 100)

3	   Indent Increment			(1 - 20)

4	   Lower case convert			(0, 1 or 2)

5	   Conditional convert character	(32 - 126)

EQ		   Quit the current edit session.

ERfile		   Open the file "file" for input.

ES n m		   Change the value of switch "n" to "m".

1	   Expand Tab with spaces		(0=NO 1=YES)

2	   Auto buffering in visual mode	(0=NO 1=YES)

3	   Start in visual mode			(0=NO 1=YES)

4	   Point past text reg. insert		(0=NO 1=YES)

5	   Ignore UC/LC distinction in search	(0=NO 1=YES)

6	   Clear screen on visual exit		(0=NO 1=YES)

7	   Reverse Upper and Lower case		(0=NO 1=YES)

ET		   Set new tab positions.

EV		   Print the VEDIT version number.

EWfile		   Open the file "file" for output.  Create Backup.

EX		   Normal exit back to CP/M after writing output file.

VEDIT - Visual Editor					      Page 65

Appendix C - Error Messages

VEDIT prints a message (on the CP/M console device) when the user

should be notified of  an unusual or  special condition.  All messages

are descriptive, and  the user  should not  normally have  to refer to

this appendix  in  order  to  understand  the  message  or error.  The

messages fall into  three categories:  fatal errors,  non-fatal errors

and other messages.  Fatal  errors  result  in  an  abort  of the disk

operation being performed and  a return  to command  mode if possible,

else a return  to  CP/M.  These  are  caused  by  certain  disk errors

described below.  The non-fatal  errors  usually  just  signify that a

typo was made or  that some  small detail  was overlooked.  These only

result in a message and the user can try again.

FATAL ERRORS

------------

OUT OF SPACE	The disk became full  before the entire output file

was  written.  As  much  of   the  output  file  as

possible was written.  Refer to the section on disk

write error recovery.

CLOSE ERROR	The  output file  could not  be closed.  This  is a

very unusual condition,  but may occur  if the disk

becomes write protected.

READ ERROR	An  error  occurred  reading  a  file.  This  error

should never  occur,  since  CP/M  itself  normally

gives an error if  there was a  problem reading the

disk.

NO DIR SPACE	There was  no directory  space left  for the output

file.  Refer to the  section  on  disk  write error

recovery.

NON-FATAL ERRORS

----------------

INVALID COMMAND	The specified letter is not a command.

CANNOT FIND...	The specified  string could not  be found.  This is

the normal return for iteration macros which search

for all occurrences of a string.

NESTING ERROR	You cannot nest macros deeper than 8 levels.

BAD PARAMETER	Something was specified wrong with your "EI", "EP",

"ES" or "ET" command.

NO INPUT FILE	There is  no input  file open  for doing  a read or

append.

VEDIT - Visual Editor					      Page 66

Appendix C - Error Messages

NO OUTPUT FILE	There is no  output file open for  doing a write, a

close or an  exit  with  the  "EX" command.  If you

have already written out the text buffer and closed

the output file, exit with the "EQ" command.

CANNOT OPEN TWO	You cannot have two  output files open and there is

already one open.  Also given if  an output file is

open at the time of an "EC" command.    Perhaps you

want to close it with the "EF" command.

BAD FILE NAME	The  file name  you gave  does not  follow the CP/M

conventions.

FILE NOT  FOUND	The file  you wanted  to open  for input  does not

exist.  Maybe you specified the wrong drive.

OTHER MESSAGES

--------------

NEW FILE	The file specified with the  EB command or with the

invocation of VEDIT did not exist on disk and a new

file has been created.  If you typed the wrong file

name, you  may want  to start  over by  issuing the

"EQ" command.

*BREAK*		The   command   execution   was   stopped   because

insufficient memory space remained  to complete the

command (I, S, G,  P and EG). For  the "I", "S" and

"EG"  commands,  as  much   text  as  possible  was

inserted.  For the "G" and "P" commands, no text at

all was copied  or inserted.   The  message is also

printed when command  execution is  stopped because

you typed [CTRL-C] on the keyboard in command mode.

QUIT (Y/N)?	This  is  the  normal  prompt  following  the  "EQ"

command.  Type "Y" or  "y"  if  you  really want to

quit and  exit  to  CP/M,  otherwise  type anything

else.

INSERT NEW DISK AND TYPE [RETURN]

This is the normal prompt  for inserting a new disk

with the "EC" command.

VEDIT - Visual Editor					      Page 67

Appendix D - VEDIT Notes

We are interested in  hearing  from  users  about  any changes or

additions they would  like to see  in VEDIT, or  even just information

about  their  application.  We  are  also  interested  in  suggestions

regarding this   manual.  Each   suggestion   will   receive  personal

attention  and  helpful  suggestions  have  a  good  chance  of  being

incorporated in future  releases, since we  are continuously expanding

the features of VEDIT.

Currently we know of the following limitations to VEDIT.

1.)   Lines longer than  258 characters,  not including  the CR,LF are

not handled well in  visual mode.  When the cursor  is on such a

line only the first  258 characters will  be displayed. The line

may be broken into smaller lines by deleting two characters with

the [Back Space], typing  [RETURN] to split the  line in two and

typing in the two deleted characters again.

##############################################################