Notes for Memory Mapped Version 1.16 of VEDIT

These notes apply predominately to the SSM VB3 video board

and describe  the  patches  and  customization  "Initialization

Bytes" needed to  fully support  the hardware  attribute cursor

and bank select features.

Just specifying the  "Initialization Bytes"  is sufficient

to get the  board up and  running, however a  very simple patch

will enable the  VB3's  attribute  cursor.  Using  the VB3 with

Bank Select requires  a  further  patch.  This  cannot be fully

described  here,  because  it  is  very  dependent  upon  other

hardware in the system.

When installing any  of the  patches, you  should refer to

the enclosed files  VEDITZM.LST (Z80) or  VEDIT8M (8080), which

contain  listings   of   the   pertinent   source   code.  When

customizing a  patched  .SET  file  you  will  get  a "CHECKSUM

ERROR".  Just type RETURN to ignore the error.

The VB3 needs to  be initialized whenever  the visual mode

is entered.  Specifically, the hardware scroll register must be

set to the last  screen line in  order to make  the Base screen

address the upper  left hand corner.  Since  VEDIT produces its

own cursor on  the  screen,  the  default  cursor  must also be

disabled.  This is done by simply moving it to a column off the

screen.  The following sequence  of  instructions  will perform

this   initialization.  They   may   be    specified   as   the

"Initialization Bytes"   during   customization,   or   may  be

permanently  patched  into  VEDIT  at  the  label  HCRSOF  (see

listing).

OUT  0E0H    ;Enable video board. (Actually OUT KSTAT)

MVI  A,17H   ;Get # Rows -1. (17H is based on 24 line screen)

OUT  0D6H    ;Init scroll register. (Actually IOBASE + 6)

MVI  A,0FFH  ;Get value to put cursor off screen.

OUT  0DCH    ;Move default cursor off screen. (Actually IOBASE + 12)

OUT  0E1H    ;Disable video board. (Actually OUT KDATA)

RET

The equivalent Initialization Bytes would be:

Pair 1 :  00      E0

Pair 2 :  17      D6

Pair 3 :  FF      DC

Pair 4 :  00      E1

Instead of using the  default hardware cursor  on the VB3,

VEDIT creates   its   own   cursor.  The   VEDIT  documentation

describes three cursor  types:  0  =  blinking  underline,  1 =

blinking reverse video block and 2 = solid reverse video block.

Of these, only type 0 will  work with the VB3.  However, VEDIT

has one additional cursor type especially  for the VB3. This is

the hardware attribute cursor,  or cursor type  "3", and may be

selected as the default  cursor during customization  or may be

changed with  the  "EP"  command.  To  implement  this hardware

attribute cursor, you  will  need  to  patch  one  byte  in the

VEDITZM.SET  or  VEDIT8M.SET   file  before   you  perform  the

Page 2

customization, or else in the  VEDIT.COM file after you perform

the customization. The byte to be  patched is in the line after

CURSO4 in the listing.  The listing  shows the instruction "DAD

B" with the  object  byte  of  "09".  This  "09"  byte has been

changed to "00"  in your ".SET"  files and needs  to be changed

back to "09" to implement the attribute cursor. (Our reason for

disabling cursor type  3  is  to  prevent  problems for non-VB3

users.  Otherwise, should a  non-VB3  user  accidentally select

cursor type 3,  VEDIT  would  begin  to  "mysteriously"  eat up

programs in  memory,  possibly  itself,  but  very  likely  the

operating system.)

The following steps show  how  to  use  DDT  to enable the

attribute cursor in the VEDITZM.SET and VEDIT8M.SET files:

B>DDT VEDITZM.SET

DDT VERS. 2.0

NEXT  PC

2E00 0100

-S2222

2222  00 09            ;DDT prompts with 00, you type 09 and <RETURN>

2223  77 .             ;You type "." and <RETURN>

-<CTRL-C>

B>SAVE 45 VEDITZM.SET  (SAVE 46 VEDITZML.SET)

B>DDT VEDIT8M.SET

DDT VERS. 2.0

NEXT  PC

3000 0100

-S244A

244A  00 09            ;DDT prompts with 00, you type 09 and <RETURN>

244B  77 .             ;You type "." and <RETURN>

-<CTRL-C>

B>SAVE 47 VEDIT8M.SET  (SAVE 48 VEDIT(ML.SET)

To implement  screen  bank  select,  you  must  patch  the

necessary code in the space provided  at the routines SCRIN and

SCROUT. In SCRIN, the code will  need to include "OUT KSTAT" to

enable the VB3, and  any necessary  code to  disable the memory

overlapping the board. In SCROUT, the code will need to include

"OUT KDATA" to  disable  the  VB3,  and  any  necessary code to

enable the memory  overlapping the  board. If  the VB3 overlaps

the main text buffer of VEDIT,  which is all memory between the

end of VEDIT and the beginning of  the CP/M BDOS, you must also

change the byte at BNKSEL from a "00" to a "01".

Please contact us  should you require  any assistance with

these patches or  with  any  other  aspect  of  VEDIT. We would

appreciate any  comments  you  might  have  regarding  the bank

select and hardware  attribute cursor,  since we  would like to

make this part  of  the  customization  as  soon  as  we find a

technique which is general  enough to work  with most available

video boards.

##