DATE : 8/21 '89

TABLE OF CONTENT

NOTES		    0-1

CHAPTER	I     INTRODUCTION

1-1 Product Overview  1-1

1.2 Name of the Parts  1-1

1.3 Specifications  1-1

(1)  Host Adapter Card  1-1

(2)  Programmer & Tester  1-2

(3)  Software	 1-2

CHAPTER	II    EQUIPMENT

2.1 Unpacking & Inspection  2-1

2.2 Computer System Required  2-1

2.3 I/O Address Setting  2-1

(1)  Host Adapter Card ( hardware )  2-1

(2)  SETUP.DAT ( software )  2-2

CHAPTER	III   INSTALLATION & TESTING

3.1 Hardware Installation	 3-1

3.2 Testing  3-1

Common User's Steps  3-2

CHAPTER	IV    EPROM/EEPROM Function description	 4-1

CHAPTER	V     MICROCOMPUTER (8751 Series)

Function description  5-1

CHAPTER	VI    MICROCOMPUTER (8748 Series ,Z8 Series..)

Function description  6-1

CHAPTER	VII   BIPOLAR PROM Function description	 7-1

CHAPTER	VIII  PAL, FPL,	GAL, PEEL, EPLD	Function description 8-1

CHAPTER	IX    DIGITAL IC & MEMORY TESTER Function description 9-1

CHAPTER	X     OTHER SOFTWARES' description 10-1

CHAPTER	XI    DAMAGE CHECK 11-1

�						       page 0-1

************************* N O T	E S ****************************

NOTE 1 :

If  there  is	 any  unclear  statement in this manual,

please  print	 the  'MANUAL.DOC'file from the	disk for

reference.  Should this manual be modified, MAMUAL.DOC

will be also corrected immediately.

NOTE 2:

First	of all please read this	manual from CHAPTER I to

CHAPTER  III	before	you start to instal and	use this

programmer.

NOTE 3:

Pin assignment of the	SOCKET as below:

pin1 for 40pin's chip - |----v----|

|	  |

pin1 for 32pin's chip - |----v----|

pin1 for 28pin's chip - |----v----|

pin1 for 24pin's chip - |----v----|

pin1 for 20pin's chip - |----v----|

pin1 for 16pin's chip - |----v----|

|	  |

|	  |

|	  |

|	  |

|---------|

|

O

NOTE 4:

With your computer turned on and off,	or your	executed

files	 replaced,  please  don't  put  the chips on the

testing  socket.  Wait  until	 the main display screen

appears.  Thus,  you	can  avoid destroying your chips

because of error operation.

NOTE 5:

In  the  programming process,	the BUSY LED will light.

please  don't  put  the  chip  on  or take it from the

socket  at this moment. The programming voltage on the

socket  may destroy the chip,	so please wait until the

BUSY LED is turned off then start next step.

NOTE 6:

After	  your	 computer  is  turned  on,  whether  the

programmer  is  connected  with  D-25	CABLE or not, it

won't  affect  the  computer  and  won't  destroy  the

programmer, too.

�                                                       page 0-2

NOTE 7:

This	programmer  owns  a warrant to succeed a 99 % up

performance.	Suppose	 your  programmer  can't perform

well,	 please	 contact  us. We will help solve problem

for  you.  Should any	problem	arise in the programming

process,  please  don't  waste too many chips to test.

First	read "DAMAGE CHECK", make sure where the problem

is, solve it then program again.

NOTE 8:

Our  company	will continue offering new softwares. If

any  chips  were  found with softwares unavailable for

programming  from  us,  please contact us. We	may have

new  software	 at  hand.  However if we don't have, we

will write it	for you	at our soonest.

�						       page 1-1

CHAPTER	I  :  INTRODUCTION

1.1 Product Overview

(1)	 This  programmer is not individually used. It has to be

connected  with	 PC  XT/AT.  With  the help of different

softwares  in  the  diskette,  it can be able to program

different ICs.

(2)	 This programmer is an Universal Programmer & Tester. It

can program ICs	as the following list:

* EPROM / EEPROM

* PAL / FPL	/ GAL /	PEEL / EPLD

* BPROM

* Microcomputer

it can also test ICs as	the following list:

* 74 series	TTL

* 40/45 series CMOS

* SRAM / DRAM

(3)	 Our  company  will  continue supplying	new softwares to

meet your requirement to program new ICs.

1.2  Name of the Parts

__________|			  __________________

|	 |__		       __|		    |

|	 |  | D-25 Connecter  |	 |    o		    |

|	 |  |=================|	 |	  _	    |

|	 |__|		      |__|	 | |	    |

----------|			 |_______|_|________|

Host Adapter Card				   |

programmer & tester

1.3 Specifications

(1)	Host Adapter Card:

The Host Adapter Card includes two types (CSA-03 ,CSA-04

).  The  function  of  these  two  types  are  same  and

transferable.

*OUTPUT	VOLTAGE	 : +5V,	regulated & current limited 1.5A

+12V, -12V.

*OUTPUT	SIGNAL : D0 -D7, A0 -A4, /IOR, /IOW. CLK, /RESET

& I/O READY.

*I/O  ADDRESS  SETUP: according	to properly adjusted I/O

PORT

a. MODEL CSA-03 can select one from the 8 sets of I/O PORTs

.(200--2FF), such as

(200-21F); (220-23F); (240-25F); (260-27F);

(280-29F); (2A0-2BF); (2C0-2DF); (2E0-2FF).

�                                                       page 1-2

b.  MODEL	 CSA-04	 can  select one from the 16 sets of I/O

PORTs(200--3FF), such as

(200-21F); (220-23F); (240-25F); (260-27F);

(280-29F); (2A0-2BF); (2C0-2DF); (2E0-2FF);

(300-31F); (320-33F); (340-35F); (360-37F);

(380-39F); (3A0-3BF); (3C0-3DF); (3E0-3FF).

* PCB SIZE: 105	mm (L) * 90 mm (W)

* CABLE	LENGTH:	100 cm (D type,	25 PINs	connector)

* WEIGHT: 250 gm.

(2) PROGRAMMER & TESTER

* Supply power

___________________________________________

|     STAND BY	       |    ACTIVE	   |

|_____________________|___________________|

|     +5v 470mA(MAX)  |   +5V	650mA(MAX) |

|_____________________|___________________|

|     +12V 150mA(MAX) |   +12V	1A (MAX)   |

|_____________________|___________________|

* Three sets of programming voltage

VL : 1.5V -- 9V 800mA (MIN)  0.15 V/div

VM : 1.5V --25V 400mA (MIN)  0.10 V/div

VH : 1.5V --25V 400mA (MIN)  0.10 V/div

* Each	pin driver power : 400 mA & 800	mA

* Standard  timer for hardware	programming:16uS,128uS &

512uS

* 2MHz	 frequency  is	supplied  for 8748, 8751, Z8 ...

micorcomputer.

* A 40-PINs Universal Narrow ZIP Socket

* Programmable Pin Driver

* Each Pin Driver has its own short circuit protected

(3)  Software

Available softwares are listed as below. New softwares

will	have  a	 continuous  increase. Please contact us

anytime for a	requiry	of programming any special IC.

Disk #1 :

README    DOC   ....	 list for software upgraded

EPP02     EXE   ....	 for EPROM & EPPROM

SETUP02   EXE   ....	 for I/O address setting

48P02     EXE   ....	 for 8741/42/48/49

51P02     EXE   ....	 for 8744/51/52

Z8P02     EXE   ....	 for Z86E11/21

BPP02     EXE   ....	 for Bipolar PROM

DMT02     EXE   ....	 test	program	 for  74/54  TTL

HEXOBJ    EXE   ....	 to  convert  .HEX  file to .OBJ

file for 8 bit HEX file.�                                                       page 1-3

HEXOBJ2   EXE   ....	 to  convert  .HEX  file to .OBJ

file for 16 bit HEX file.

DASM48    EXE   ....	 dis-assembler for 8748/49

40/45 CMOS, SRAM & DRAM

Disk #2 :

PAP02     EXE   ....	 for PAL(MMI,NS,TI,SIGNETICS)

PAPA02    EXE   ....	 for PAL(AMD/CYPRESS/TICPAL)

PARTS02   LST   ....	 parts	list  for programmable &

testable ICs

DASM51    EXE   ....	 dis-assembler for 8751/52

IOCHK02   EXE   ....	 I/O address check

22G10-02  EXE   ....	 for  22G10  ( CYPRESS )

PALFORM (DIR)   ....	 fuse map for PAL's

Disk #3 :

FPL02     EXE   ....	 for Signetics PLHS 153/173 FPLA

A18P8-02  EXE   ....	 for AmPAL18P8

PEEL02    EXE   ....	 for PEEL(AMI/GOULD/ICT/HYUNDAI)

S-GAL02   EXE   ....	 for PLC16V8,18V8Z,20V8 & 22V8Z

(Signetics)

GAL02     EXE   ....	 for GAL 16V8/20V8(LATTICE /NS /

SGS /VLSI)

22V10-02  EXE   ....	 for  GAL  22V10  ( AMD /  ATMEL

/ CYPRESS / MMI / TI / TICPAL )

TEST02    EXE   ....	 voltage adjust & hardware check

for programmer

MANUAL    DOC   ....	 user's manual

�						       page 2-1

CHAPTER	II   EQUIPMENT

2.1  Unpacking & Inspection

Becareful to unpack the package and check details as below:

* Host Adapter Card

* Connecting Cable

* Programmer Unit

* Utility Disk

* User's Menual

(Please	 check in detail to see	if you got everything in

it.  Should  there  be	any  damage  or	shortage, please

contact	the dealer for replacement.)

2.2  Computer System Required

* IBM PC XT/AT or compatible system (Min. 384K RAM)

* PC DOS V2.0 later or compatible DOS

2.3  I/O ADDRESS SETTING

You  can  skip	over  this section and direct to CHAPTER

III.  If  you meet problem when	you execute CHAPTER III,

you can	return to this section for answer.

I/O  address  setting  includes	 two  parts:  one is the

setting	 on  Host  Adapter  Card  and  the  other is the

setting	 on  the  software-SETUP.DAT.  Anyway  these two

settings must be same.

(1) I/O address setting on	the Host Adapter Card:

There are 2 different models for the Host Adapter Card.

They  have  the  same	function and any one of	them can

connect  with	the  programmer	to be used but their I/O

address settings are different.

a. Model CSA-03 model :

There	is  one	 set of	JUMPER (JP1) on	the Host Adapter

Card.	The relation between the JUMPER	position and the

system	I/O address (200H-2FF) as below:

�                                                       page 2-2

________________________________________________

|	  JUMPER (JP1)	 I/O ADDRESS		    |

|	      1		 200-21F		    |

|	      2		 220-23F		    |

|	      3		 240-25F		    |

|	      4		 260-27F		    |

|	  **  5	 *****	 280-29F  ** preselected    |

|	      6		 2A0-2BF     in	our factory |

|	      7		 2C0-2DF		    |

|	      8		 2E0-2FF		    |

|______________________________________________|

b. Model CSA-04:

There	are two	sets of	JUMPERs	(JP1, JP2). The	relation

between  the JUMPER position and the system I/O address

(200H--3FF) is	as below:

________________________________________________

| JP2 (1--2 3 ) ->	I/O ADDRESS 200	--2FF	     |

|	    JUMPER (JP1)    I/O	ADDRESS		     |

|		1	200-21F			     |

|		2	220-23F			     |

|		3	240-25F			     |

|		4	260-27F			     |

|	   **	5  ***	280-29F	** preselected	     |

|		6	2A0-2BF	   in our factory    |

|		7	2C0-2DF			     |

|		8	2E0-2FF			     |

|_______________________________________________|

|	JP2 (1 2-3 ) ->	I/O ADDRESS  300-3FF	     |

|		1	 300-31F		     |

|		2	 320-33F		     |

|		3	 340-35F		     |

|		4	 360-37F		     |

|		5	 380-39F		     |

|		6	 3A0-3BF		     |

|		7	 3C0-3DF		     |

|		8	 3E0-3FF		     |

|_______________________________________________|

*  As	finished  , the	setting	way of JP1 & JP2 is also

the I/O address on the programmer.

(2) SOFTWARE SETTING

With  every  kind  of	programming  softwares executed,

softwares  will  call to the I/O address in SETUP. DAT,

therefore  the	I/O address on the Host	Adapter	Card and

in SETUP.DAT must be same.

Execute  SETUP02.EXE to select	the JUMPER's setting way

on  the Host Adapter Card. After executing, I/O address

will be saved in SETUP.DAT file automatically.

�                                                       page 2-3

For example:

A> SETUP02  <enter>

I/O address setup	for Host Adaptor Card	  V2.0

(C) JAN 25	'89

________________________________________________

|Current I/O address is:	280-29FH	     |

|	 JP1 at	PIN 5 .	JP2 at PIN 1&2		     |

|______________________________________________|

Change	I/O address

(A) For no JP2 on	the card, or JP2 is pin	1 & 2

shorted

(B) For JP2 is pin 2&3 shorted

<ESC> Exit & Save I/O address

________________________________________________

| I/O address is from 200H	to 2FFH		     |

|	 The jumper of JP1 at which position(1-8) :  |

|_______________________________________________|

REMAKE  :	If the Host Adapter Card is Model CSA-03, please

select	"A". If	the Host Adapter Card  is  Model

CSA-04, you may select "A" or "B".  The  relation

with JUMP 2 is as below:

1.	Please	choose	"A"  as	 JP2(1--2 3) on	the Host

Adapter Card. (200H-2FFH)

2.	Please	choose	"B"  as	 JP2(1 2--3) on	the Host

Adapter	Card.  (300H-3FFH)

<ESC>  :  save file, return to DOS

*	As  finished  in our factory, the I/O address of

the  software will be preselected 280H--29FH.

For example:

A>	 TYPE  SETUP.DAT

A5

||

||__ JP1 :	PIN 5 (the 5th set)

|___ JP2 :	(1--2 3)       ie : 280H--29FH

A>	 TYPE  SETUP.DAT

B6

||

||__ JP1 :	PIN 6 (the 6th set)

|___ JP2 :	(1 2--3)       ie : 3A0H--3BFH

�						       page 3-1

CHAPTER	III  INSTALLATION & TESTING

3.1 Hardware Installation

1.	Please turn off	the power of your computer.

2.	 Plug  the  Host  Adapter  Card	 into  any  slot on your

computer.

3.	Use the	cable to connect the Host Adapter Card with your

programmer.

3.2 Testing

1.	Power on your PC and load DOS.

2.	 After	A>  displayed on the screen, execute IOCHK02.EXE

program.  (There must be SETUP.DAT. in the disk	files so

as  to test whether the I/O ADDRESS on the programmer is

coordinative with the other cards)

3.	 If  an	error occurs, please refer to section 2.3, " I/O

ADDRESS  SETTING  ".  Try  to  modify the I/O address on

software & hardware then start again from section 3.1 to

test.

4.	If no error occurs, the	testing	is O.K.

�						       page 3-2

======================COMMON USER'S STEPS======================

1. Check up the	filename and the manufacturer's name from the IC

LIST	for the	IC which you are going to program.

REMARK  :  Different	 types	of  ICs	 may  come from	the same

manufacturer,  therefore,	you  have  to  check  up

PARTS02.LST first.

For	example: MMI PAL 16L8B-2 uses the filename of PAP02.EXE,

The manufacturer is MMI-A TYPE.

2. Execute this	file. (simply enter the	filename)

For example:	A>PAP02	 <enter>

3.  After  main	 menu  appears,	 press	key  "M"  to  choose the

manufacturer. For example:

PAL SOFTWARE	 V3.0		    *MFG.:  NS	      CHECK SUM

*TYPE: NONE-TYP    =:0

MAIN MENU:		    *FUSE MAP: NONE-MAP

============================	  ______________________________

1. DIR				 |  MANUFACTURER:		|

2. LOAD	FUSE MAP FROM DISK	 |  1.MMI (A TYPE) 4.MMI(B TYPE)|

3. SAVE	FUSE MAP TO DISK	 |  2. NS	   5.TI(B TYPE)	|

4. EDIT	FUSE MAP		 |  3.TI (A TYPE)		|

M. MANUFACTURER			 | <ESC>back to	main manu.	|

T. TYPE				 |				|

B. BLANK CHECK			 |______________________________|

P. PROGRAM   A.	AUTO

R. READ	     V.	VERIFY

S. SECURITY FUSE BLOW

Q. QUIT

*	Press  key  '1'	 to  choose  MMI-A TYPE	and press ESC to

return to main manu.

4. Press key 'T' to choose TYPE.

For example:

�                                                       page 3-3

PAL SOFTWARE	V3.0		*MFG.: MMI (A TYPE)   CHECK SUM

*TYPE: NONE - TYP	=:0

*FUSE MAP: NONE	MAP

MAIN MANU:		 ________________________________________

======================	|   TYPE: 24 pins			|

1. DIR			|1:12L10  A:20X8   L:20R4A/A-2/B/B-2/D	|

2. LOAD	FUSE MAP FROM	|2.14L8	  B:20X8A  M:20L8-10		|

DISK			|3.16L6	  C:20X4   N:20L8A/A-2/B/B-2/D	|

3. SAVE	FUSE MAP TO DISK|4.18L4	  D:20X6            		|

4. EDIT	FUSE MAP	|5.20L2	  E:20X8A              		|

M. MANUFACTURER		|6.20C1	  F:20L4                        |

T. TYPE			|7.20L10  G:20R8-10			|

B. BLANK CHECK		|8.20X10  H:20R8A/A-2/B/B-2/D		|

P. PROGRAM   A.AUTO	|9.20X10A I:20R6-10			|

R. READ	     V.VERIFY	|	  J:20R6-10			|

S. SECURITY FUSE BLOW	|	  k:20R4-10			|

Q. QUIT			|					|

|    <ESC> back	to main	manu.		|

|    <PGUP>next	page			|

|    SELECT NUMBER?			|

|_______________________________________|

#	Press key 'J' to choose	16L8B-2/-4 and press ESC back to

main manu.

5.  To	program	 the  data from	the disk files to the blank PAL,

execute  function  2	 (LOAD	FUSE MAP FROM DISK), followed by

entering the	path and the filename. For example:

PAL SODTWARE  V3.0	  *MFG.: MMI (A	TYPE) CHECK SUM

*TYPE: 16L8B-2/-4	=:0

MAIN MANU:		  *FUSE	MAP: A:TEST.JED

==============================	 ___LOAD FUSE MAP:_____________

1. DIR				 |			      |

2. LOAD	FUSE MAP FROM DISK	 |  Enter Fuse map file	name  |

3. SAVE	FUSE MAP TO DISK	 |  <ESC> back to main manu.  |

4. EDIT	FUSE MAP		 |			      |

M. MANUFACTURER			 |   A:	TEST.JED	      |

T. TYPE				 |____________________________|

B. BLANK CHECK

P. PROGRAM	A. AUTO

R. READ		V. VERIFY

S. SECURITY FUSE BLOW

Q. QUIT

To  program  data  from	 the PAL MASTER	to the blank PAL,exectue

function R(READ). For example:

�                                                       page 3-4

PAL SOFTWARE	 V3.0	       *MFG.: MMI (A TYPE)  CHECK SUM

*TYPE: 16L8B-2/-4     =:1560

*FUSE MAP: A:TEST.JED

MAIN MANU:	      _READ : __________________________

===========================  |	 Ready to read (Y/ESC)?	       |

1. DIR			     |	 Reading now...		       |

2. LOAD	FUSE MAP FROM DISK   |	 Read complete		       |

3. SAVE	FUSE MAP TO DISK     |_________________________________|

4. EDIT	FUSE MAP

M. MANUFACTURER

T. TYPE

B. BLANK CHECK

P. PROGRAM

R. READ

S. SECURITY FUSE BLOW

Q. QUIT

6. Put the blank IC on the testing socket, press P (function P),

and press Y to start	to program.

PAL SOFTWARE	 V3.0		 *MFG.:	MMI (A TYPE)	CHECK SUM

*TYPE:	16L8B-2/-4	  =:1560

*FUSE MAP: NONE-MAP

MAIN MANU:	      ____PROGRAM : ___________________

===========================   |	Blank check or not (Y/N/ESC)?  |

1. DIR			      |	Checking now ...	       |

2. LOAD	FUSE MAP FROM DISK    |	Blank check ok.		       |

3. SAVE	FUSE MAP TO DISK      |				       |

4. EDIT	FUSE MAP	      |	Blowing	now...		       |

M. MANUFACTURER		      |	Verify ok.		       |

T. TYPE			      |________________________________|

B. BLANK CHECK

P. PROGRAM    A. AUTO

R. READ	      V, VERIFY

S. SECURITY FUSE BLOW

Q. QUIT

�						       page 4-1

CHAPTER	IV  EPROM / EEPROM

========================= FUNCTION DESCRIPTION ==================

** Files required :

1. EPP02.EXE : the main	program	to be executed.

2.  EPP.DAT  :	to  save the previous running data after

function	Q: Quit	was executed.

3. SETUP02.EXE

SETUP.DAT

(This file is for I/O address setting)

REMARK  : Please place above files	in the same working disk

drive.

***  The  function  menu  will be automatically	displayed on the

screen as showed below after EPP02.EXE is executed.

E(E)PROM  PROGRAMMER  V3.0	     *MFG.:AMD	  *ZIP.	:1

*TYP.:2764A  *PROG.:intelligent

MAIN	 MANU:		     *VPP.: 12.5V *VCC.	: 6.0V

==================================

1. DIR

2. LOAD	OBJ FILE TO MEMORY BUFFER

3. SAVE	MEMORY BUFFER TO DISK

4. DEBUG MEMORY	BUFFER

5. GANG	SIZE

6. PROGRAMMING ALGORITHM

7. SET MEMORY BUFFER SIZE

M. MANUFACTURER

T. TYPE

B. BLANK CHECK

P. PROGRAM	A. AUTO

R. READ		V. VERIFY

C. COMPARE	D. DISPLAY & EDIT

Q. QUIT

SELLECT	WHICH NUMBER ?

REMARK 1 :	Above right angle

*	MFG.  stands  for manufacturer(refer to	function

M)

*	TYP.  stands for TYPE (refer to	function T)

*	VPP.  stands for the programming voltage. (refer

to function T)

*	ZIP.  stands   for  the	 number	 of  programming

socket. (refer to	function 5)

*	PROG. stands  for  the	programming  algorithm &

time (refer to function 6)

*	VCC.  stands for VCC voltage in	programming

�                                                       page 4-2

REMARK 2 :	 Above	data  about  MFG.  TYP.	... etc. will be

saved   into  EPP.DAT  file  when  you  execute

function  Q, and will be read from EPP.DAT when

you  run  EPP02.EXE program next time,	and will

be automatically selected as the same status as

the last time.	(MFG. TYP. ... etc.)

REMAKE 3 :	After executing	EPP02.EXE, PC will open	a 64K or

128K  memory  space  as	 MEMORY	 BUFFER	for user

(refer	to  function  7). The address for MEMORY

BUFFER	is  selected from 00000H--0FFFF	(64K) or

00000H--1FFFF  (128K).	Data read from the EPROM

or the DISK will be saved in MEMORY BUFFER.

*** FUNCTION 1 : DIR

To list the files directory of	the disk.

(under	the same DOS system)

FORMAT : [d:] [path] [filename {.ext}] [/p] [/w]

<ESC> back to main menu.

REMARK : COMMAND.COM  must be placed in	drive A	so as to

execute DOS COMMAND.

*** FUNCTION 2 : LOAD OBJ FILE INTO MEMORY BUFFER

To load .OBJ file from	the disk into the memory

buffer,  enter	 the path, followed by .OBJ file

name  and  the	buffer starting	address	or press

<ESC> to return to main menu.

FORMAT : Enter file name to be loaded :

[d:] [path] [filename,OBJ]

Enter buffer starting address:

[n] ----  Usually "0" is input	and started from

the	first  address in MEMORY BUFFER.

or press <ESC>	to return to main menu.

REMARK : Please	load the OBJ CODE file.	If the .HEX file

was loaded, please use	 HEXOBJ02.EXE to convert

it into .OBJ file before you input data.

*** FUNCTION 3 : SAVE MEMORY BUFFER TO DISK

To  save  data	 from  the  memory buffer to the

disk,	enter  the  path,  followed by .OBJ file

name,	the  buffer  starting and end address or

press <ESC> to	return to main menu.

FORMAT : Enter file name to be saved:

[d:] [path] [filename.OBJ]

Enter buffer starting address:

�                                                       page 4-3

[n1] ---- the	buffer	starting  address  to be

saved

Enter buffer end address:

[n2] ---- the buffer end address to be	saved

or <ESC> back to main menu.

REMARK : Above data will be saved in OBJ CODE format.

*** FUNCTION 4 : DEBUG MEMORY BUFFER

To debug and modify your memory buffer.

REMARK : Please	 load  DEBUG.COM  into	the working disk

driver	and press any key to start (see	DEBUG in

MS DOS). The display screen looks like	this:

First 64K memory buffer starting address at

4356:0000

|___	data segment

Second	64K memory buffer starting address at

5356:0000

|___	data segment

First enter RDS to change data	segment

-RDS

-DS: 1245

-4356	---- input  data  segment  of the memory

buffer start address

*** FUNCTION 5 : GANG SIZE

To select the number of SOCKET	in a working

batch ( 1-4 SOCKET(S)).

REMARK : If  you  want	to expand it to	4 , you	shall be

asked to buy additional 4 ZIP sockets.

*** FUNCTION 6 : PROGRAMMING ALGORITHM

To select the type of the programming algorithm.

REMARK : The   programming  algorithm  is  automatically

selected  when	 you change the	MFG. or	TYP. but

you  can change it by yourself	only you make it

in accordance with the	IC programming rules.

*** FUNCTION 7 : SET MEMORY BUFFER SIZE

To set  memory  buffer size, you may select 64K

�                                                       page 4-4

or  128K.  As executing this function to change

memory buffer size, the computer will return to

DOS   automatically .  At  this  moment  please

execute EPP02.EXE program again.

For example : EPROM	128K*8	such as	NEC 27C1000, 27C1001 ...

etc. must have	128K MEMORY SIZE.

REMARK 1 : As  function  Q is executed, MEMORY BUFFER SIZE

(64K or 128K) will be save in EPP.DAT file.

REMARK 2 : If  the 128K is selected, PC's MEMORY SIZE must

be  at	 least	384K  up  so as	to execute EPP02

.EXE. If EPP02.EXE fails to be	executed, please

erease	EPP.DAT	file then try EPP02.EXE	again.

*** FUNCTION M : MANUFACTURER

To select the manufacturer of the chips.

* Please	 select	the correct manufacturer because

the  programming algorithm of each manufacturer

is quite different.

* If  IC	manufacturer is	not found, please choose

the first set "DON'T CARE".

* If  DON'T CARE is selected, you must assure the

TYPE  and  the	 programming  voltage  .  If the

programming  voltage  is unknown, please select

the  low  voltage  to test. If	failed,	try high

voltage.

*** FUNCTION T : TYPE

To  select  TYPE  and	programming voltage, use

keys <1><2>...<A><B>...etc.

CAUTION : Please	 select	the exactly correct TYPE and the

programming voltage.

* The	programming    algorithm   of	National

Semiconductor 27CP64 ,	27CP128	is comparatively

special    and	  different   from   the   other

manufacturers' 2764 , 27128 .

* There	are  two  different PIN	structure for 32

PINs  IC  and	the  IC	 part  number  from each

manufacturer is quite different.

PINs A : ( A16=pin2 , /OE=pin24 )

DON'T CARE : 27010

AMD : 27C010

FUJITSU : 27C1001

HITACHI : 27C101

�                                                       page 4-5

INTEL : 27010 , 27C010

MITSUBISHI : 27C101

TOSHIBA  : 27C1000

PINs B : ( A16=pin24 , /OE=pin2 )

DON'T CARE : 27100

FUJITSU : 27C1000

HITACHI  : 27C301

MITSUBISHI : 27C100

NEC : 27C1000

TOSHIBA : 27C1001

*** FUNCTION B : BLANK CHECK

To  check  whether  the chips are blank or not.

The length is decided by the section address. (

the  section  address described below)	If there

is data in the	first address, the error will be

displayed.

For example :

NO.1   ERROR   AT	 138

< The first EPROM at address 0138 is not blank)

<Y> : Start to check

<C> : to change section address

<ESC>: back	to main	menu

Section Address :

CHIP STARTING ADR  : the  starting  address  of

the IC's on SOCKET.

CHIP	   END ADR  : the  end	address	 of  the

IC's on SOCKET.

BUFFER	STARTING ADR: MEMORY  BUFFER's  starting

address.

BUFFER	CHECK SUM   : accumulated by BYTE value

BUFFER  CHECK SUM's expression is accounted from the BUFFER

STARTING  ADDRESS,	 its  length  =	 (CHIP	END ADR) - (CHIP

STARTING ADR)

REMARK : Sometimes  a  damaged	IC may be mistaken for a

blank IC by doing BLANK CHECK.

*** FUNCTION P : PROGRAM

To  program  the  chip	 with  the data	from the

memory	  buffer   to	EPROM.	It  will  VERIFY

automatically	 after	 finishing  program  and

display  the result. The address and the length

are decided by	section	address.

�                                                       page 4-6

<Y>	: Start	to program.

<E>	: to program even bytes	only

<O>	: to program odd bytes only

<C>	: to change the	address

<ESC>: back to main menu.

For example : Press	'O'  to	 program  data	addressed at odd

bytes	only  in  the  memory  buffer  as showed

below:

_________________________________________

|  MEMORY BUFFER   |	EPROM		|

|  ADDRESS  DATA   |   ADDRESS	DATA	|

|__________________|____________________|

|  0000	  55	   |   0000  55		|

|  0001	  38	   |   0001  AA		|

|  0002	  AA	   |   0002  12		|

|  0003	  49	   |	  :		|

|  0004	  12	   |	  :		|

|  0005	  6C	   |	  :		|

|     :		   |	  :		|

|_____:____________|______:_____________|

Press	'E'  to	 program  data addressed at even

bytes	only  in  the  memory  buffer  as showed

below:

_________________________________________

|   MEMORY BUFFER   |	 EPROM		|

|  ADDRESS  DATA	   |   ADDRESS	DATA	|

|___________________|____________________|

|   0000	 55	   |   0000 38		|

|   0001	 38	   |   0001 49		|

|   0002	 AA	   |   0002 60		|

|   0003	 49	   |	 :		|

|   0004	 12	   |	 :		|

|   0005	 6C	   |	 :		|

|      :		   |	 :		|

|___________________|____________________|

*** FUNCTION A : AUTO

To program automatically.

*** steps executed	automatically

1. BLANK CHECK.

2. PROGRAM.

3. VERITY.

�                                                       page 4-7

*** FUNCTION R : READ

To  read  the	data on	the chip into the memory

buffer	.

<Y> : start to read.

<E> : to  read data and save	it at even bytes only in

the memory buffer

<O> : to  read  data	and save it at odd bytes only in

the memory buffer

<C> : to change the address

<ESC> : back to main menu.

REMARK : After completing reading data from EPROM to the

memory	buffer,	first you have to assure whether

the  CHECK  SUM is correct or not. If it is not

correct,  it means that the data read is wrong.

If  the original CHECK	SUM in EPROM is	unknown,

please	 check whether the content of the memory

buffer	 is correct or not by using the	function

D.

*** FUNCTION V : VERIFY

To  verify  the  data in the chips with that in

the memory buffer.

<Y> : start to verify

<E> : to  verify  with data at even bytes only in the

memory	buffer

<O> : to  verify  with  data	at odd bytes only in the

memory	buffer.

<C> : to change the address

<ESC> : back to main menu.

*** FUNCTION C : COMPARE

To  compare  the  data	in the chip with that in

the  memory buffer. The difference is displayed

in the	format:

CHIP	ADDR : CHIP DATA - ( BUFFER ADDR : BUFFER DATA )

As  the  difference is	displayed, you may press

<CTRL-S>  to  hold the	display	or press any key

to  continue  or  press <ESC> to return to main

menu.

<Y> : start to compare

<E> : to  compare with data at even bytes only in the

memory	buffer

<O> : to  compare  with data	at odd bytes only in the

memory	buffer

�                                                       page 4-8

<C> : to change the address

<ESC> : back to main menu

*** FUNCTION D : DISPLAY & EDIT

To  display  and edit the content of the memory

buffer	 directly.  DEBUG.COM. is not neccessary

for  this  function.  Press Q to return to main

menu.

*** FUNCTION Q : QUIT

To return to DOS and save the current status of

the   chips  (MFG.  TYP.  ...	etc.)  into  the

EPP.DAT.  Next	 time when you run the EPP02.EXE

program, the same IC type as the last time will

be automatically selected.

�						       page 5-1

CHAPTER	V   MICROCONTROLLER ( 8751 Series )

========================= FUNCTION DESCRIPTION ==================

** Files required :

1. 51P02.EXE : the main	program	to be executed.

2. 51P.DAT   : to  save	 the previous running data after

function	Q: Quit	was executed.

3. SETUP02.EXE

SETUP.DAT

(This file is for I/O address setting)

REMAKE : Please  place Above files	in the same working disk

drive.

***  The  function  menu  will be automatically	displayed on the

screen as showed below after 51P02.EXE is executed.

8751  PROGRAMMER  V3.0     *MFG.: AMD	  *ZIP	: 1

*TYP.: 8751H  *PROG.: intelligent

MAIN MANU 		    *VPP.: 21.0V  *VCC.	: 5.0V

==================================

1. DIR

2. LOAD	OBJ FILE TO MEMORY BUFFER

3. SAVE	MEMORY BUFFER TO DISK

4. DEBUG MEMORY	BUFFER

5. GANG	SIZE

6. PROGRAMMING ALGORITHM

7. ENCRYPTION TABLE SETTING

M. MANUFACTURER

T. TYPE

B. BLANK CHECK

P. PROGRAM	 A. AUTO

R. READ		 V. VERIFY

C. COMPARE	 D. DISPLAY & EDIT

S. SECURITY BIT	PROGRAMMING

E. ENCRYPTION TABLE PROGRAMMING

Q. QUIT

SELLECT	WHICH NUMBER ?

REMAKE 1 :	Above right angle

*	MFG.  stands for Manufacturer (refer to	function

M)

*	TYP.  Stands for TYPE (refer to	function T)

*	VPP.  Stands for the programming voltage. (refer

to function T)

*	ZIP.  stands for the number of SOCKET. (refer to

function 5)

*	PROG. stands  for  the	programming  algorithm &

time (refer to function 6)

*	VCC.  stands for VCC voltage in	programming.

�                                                       page 5-2

REMARK 2 :	Above  data  about  MFG.  TYP.	... etc. will be

saved  into  51P.DAT  file  as the function Q is

executed.  The	data  will  be read from 51P.DAT

file  as  51P02.EXE  is	exected	next time and be

automatically  set  the	 same status as	the last

time.

REMARK 3 :	After  51P02.EXE is executed, the PC will open a

64K  memory space as MEMORY BUFFER for user. The

address	 for  MEMORY BUFFER is set from	0000H --

FFFF (64K). Data read from the EPROM or	the DISK

will be	saved in MEMORY	BUFFER.

*** FUNCTION 1 : DIR

To list the files directory of	the DISK.

(under	the same DOS system)

FORMAT : [d:] [path] [filename {.ext}] [/p] [/w]

or <ESC> back to main menu.

REMARK :  COMMAND.COM  must  be put in drive A  so as to

execute DOS COMMAND.

*** FUNCTION 2 : LOAD OBJ FILE INTO MEMORY BUFFER

To load .OBJ file from	disk into MEMORY BUFFER.

You  may specify the path, .OBJ file file name,

the  buffer  starting	address	 to be loaded or

press <ESC> to	return to main menu.

FORMAT : Enter file name to be loaded:

[d:][path][filename .OBJ]

Enter buffer starting address:

[n]  ----  Usually  '0'  is  input  and started

from  the  first  address  in MEMORY

BUFFER.

or press <ESC>	to return to main menu.

REMARK : Please	load the OBJ CODE file.	If the .HEX file

was loaded, please use	the HEXOBJ02.EXE file to

convert  it  into  .OBJ  file	before you input

data.

*** FUNCTION 3 : SAVE MEMORY BUFFER TO DISK

To  save  data	from MEMORY BUFFER into	the disk

file.	You  may  specify  the	path,  the  .OBJ

filename,  the	 buffer	starting and end address

to  be	 saved	or press <ESC> to return to main

menu.

�                                                       page 5-3

FORMAT : Enter file name to be saved:

[d:][path][filename.OBJ]

Enter buffer starting address :

[n1] ---- the starting	address

Enter buffer end address :

[n2] ---- the end address

or press <ESC>	to return to main menu

REMARK : Above data will be saved with OBJ CODE	format.

*** FUNCTION 4 : DEBUG MEMROY BUFFER

To debug and modify your memory buffer.

REMARK : Please	 place	DEBUG.COM  in  the  working disk

drive	and press any ket to start ( refer DEBUG

in DOS). The display on the screen shows:

Memory	buffer starting	address	at 4356:0000

|

|__ data

segment

<First	enter RDS to change data segment>

--RDS

--DS: 1245

--4356	 ---- input   data   segment  of  memory

buffer start address

*** FUNCTION 5 : GANG SIZE

To  select  the  number  of SOCKET in a working

batch ( 1-4 SOCKET(S)).

REMAKE : If  you  want	to  expand  it to 4 SOCKETS, you

shall be asked	to buy additional 4 ZIP	sockets.

*** FUNCTION 6 : PROGRAMMING ALGORITHM

To select the type of the programming algorithm.

REMAKE : The   programming  algorithm  is  automatically

selected  when	 you change the	MFG. or	TYP. but

you  can  also	 change	 it by yourself	only you

make  it  in accordance with the IC programming

rules.

*** FUNCTION 7 : ENCRYPTION TABLE SETTING

To  set 32 byte encryption table for protection

.  You	 may enter into	another	subfunction menu

as below in executing function	7 .

�                                                       page 5-4

ENCRYPTION TABLE :

FF FF FF FF FF FF FF FF	FF FF FF FF FF FF FF FF	=>   contect  of

FF FF FF FF FF FF FF FF	FF FF FF FF FF FF FF FF	     32	byte

encryption

table.

ENCRYPTION  TABLE  MENU	 :

1. Edit	encryption table .

2. Load	encryption table from disk .	=>  subfunction	 menu

3. Save	encryption table to disk .

4. Enable/Disable encryption table .

SELECT WHICH ONE

or PRESS <ESC> TO MAIN MENU ?

*** ENCRYPTION TABLE DISABLE ***   ==> show  whether  encryption

table  is  programmed  or

not .

* SUBFUNCTION 1	: Edit encryption table

To edit , the	32 byte	encryption table will be

displayed  in	the top	two lines on the screen.

As  soon  as	every  one  byte  is edited, the

encryption  table  will  follow  to  change  .

Remember  to	save  encryption table into disk

after	 completing  editing. ( use subfunction

3)

* SUBFUNCTION 2	: Load encryption table	from disk

To read the edited encryption	table from disk.

* SUBFUNCTION 3	: Save encryption table	to disk

To save the edited encryption	table to disk.

* SUBFUNCTION 4	: Enable/Disable encryptiion table

To   set   programmable   or	non-programmable

encryption  table  .	Every time this	function

key  is  pressed  ,  the opposite setting will

show on the screen,

*** ENCRYPTION TABLE ENABLE ***

OR	  *** ENCRYPTION TABLE DISABLE ***

If  the encryption table is set programmable ,

as  soon  as	you return to main menu	, on the

bottom  of  the  screen  will	 show message as

below	:

***	ENCRYPTION TABLE ENABLE	***

REMAKE	: 1. 8751H/8752H  have only one	Security fuse to

prevent inside ROM	from being read. Besides

the  Security  fuse  ,  87C51  /  87C51FA /

�                                                       page 5-5

87C252  have  the	32 byte	encryption table

programming.

2. After  the	 Security fuse is programmed not

only  the inside ROM can't be read but also

the outside ROM program can not be	executed

.	If  only the 32	byte encryption	table is

programmed	, the outside ROM program can be

executed  and  the	 inside	ROM can	be read.

However,  if  original  32	 byte encryption

table  is	unknown,  the data read	from the

inside ROM	would be in error . You	may gain

two   functions  described  below	 if  you

program encryption	table only.

a. You may use Verify	mode to	know whether the

inside  program  is  corrent  or  not.  (Of

course   you  have	 to  know  the	original

setting of	the 32 byte encryption table )

b. You  can  protect	the  inside program from

being copied.

*** FUNCTION M : MANUFACTURER

To select the manufacturer of the chips.

*  Please select	the correct manufacturer because

the  programming algorithm of each manufacturer

is quite different.

*  If  IC's  manufacturer  is  not  found .please

select	the first set "DON'T CARE".

*  If DON'T CARE is selected, you must assure the

TYPE	and  the  programming  voltage.	 If  the

programming  voltage  is unknown, please select

the  low  voltage  to test. If	failed,	try high

voltage.

*** FUNCTION T : TYPE

Use keys <1><2>...<A><B>...etc, to select TYPE.

The  correct  programming voltage and algorithm

will be automatically set as you select TYPE .

CAUTION : Please	  don't   feel   free   to   change  the

programming voltage and algorithm.

*** FUNCTION B : BLANK CHECK

To  check  whether  the chips are blank or not.

The length is decided by the section address. (

�                                                       page 5-6

the  section address is described as below). If

there is data in the first address, the display

will show error.

For example:

NO. 1    ERROR   AT 138

(The first address at 0138 is not blank)

<Y> : start to check

<C> : to change the address

<ESC> : back to main menu

Section address : CHIP STARTING ADR.:   the	starting address

of  the  chip  on the

socket

CHIP    END   ADR.:   the	end  address  of

the	 chip	on   the

socket

BUFFER STARTING ADR.: the	starting address

of memory buffer

BUFFER CHECK SUM  :   accumulated	by  BYTE

value.

< Above is presented by ASCII >

BUFFER  CHECK  SUM's expression is accounted from the BUFFER

STARTING  ADDRESS,	its  length  =	(CHIP END ADR )	- ( CHIP

STARTING ADR.)

REMARK : Sometimes a damaged chip will be mistaken for a

blank chip by using BLANK CHECK.

*** FUNCTION P : PROGRAM

To  program  the chip with data from the memory

buffer	 to  EPROM. It will VERIFY automatically

after finishing program and display the result.

The  address  and  the	 length	 are  decided by

section address.

<Y> : start to program

<C> : to change the address

<ESC>: back to main menu

*** FUNCTION A : AUTO

To program automatically.

** steps executed	automatically :

1. BLANK CHECK

2. PROGRAM

3. VERIFY

4. SECURITY FUSE BLOW

�                                                       page 5-7

REMAKE : if  the function 7 ***	ENCRYPTION TABLE ENABLED

***	is    set,   it	  will	 be   programmed

automatically.

*** FUNCTION R : READ

To read the data on the chip into memory buffer.

<Y> : start to reat

<C> : to change the address

<ESC> : back to main menu

REMAKE 1 : If  the encryption table is programmed	into the

chip  ,  the  original	32 byte	encryption table

and  the  ***	ENCRYPTION TABLE ENABLE	*** must

have  been  set  by  using  function  7  before

reading , so the data read will be correct.

REMAKE 2 : After	completing  reading  data  from	EPROM to

MEMORY	BUFFER,	first you have to assure whether

the  CHECK  SUM is correct or not. If it is not

correct,  it means that the data read is wrong.

If  the original CHECK	SUM in EPROM is	unknown,

please	 check whether the content of the MEMORY

BUFFER	 is correct or not by using the	function

D.

*** FUNCTION V : VERIFY

To  verify  the  data in the chips with that in

the memory buffer.

<Y> : start to verify

<C> : to change the address

<ESC> : back to main menu.

REMAKE : If  the encryption table is programmed	into the

chip  ,  the  orginial	32 byte	encryption table

and  ***  ENCRYPTION TABLE ENABLE *** must have

been set by using function 7 before verifying ,

so the	verify data will be correct.

*** FUNCTION C : COMPARE

To  compare  the  data	in the chip with that in

the  memory buffer. The difference is displayed

in the	format:

CHIP	ADDR : CHIP DATA - ( BUFFER ADDR : BUFFER DATA )

As  the  difference is	displayed, you may press

<CTRL-S>  to  hold the	display	or press any key

�                                                       page 5-8

to  continue  or  press <ESC> to return to main

menu.

<Y> : start to compare

<C> : to change the address

<ESC> : back to main menu

REMAKE : If  the encryption table is programmed	into the

chip  ,  the  orginial	32 byte	encryption table

and  the  ***	ENCRYPTION TABLE ENABLE	*** must

have  been  set  by  using  function  7  before

comparing , therefore the compared data will be

correct.

*** FUNCTION S : SECURITY BIT PROGRAMMING

To  program  security bit to protect inside ROM

from being read . If the U.V. eraser is used to

move  the  protection of the Security bit , the

inside	ROM programs will be erased , too.

*** FUNCTION E : ENCRYPTION TABLE PROGRAMMING

To   program	32  byte  encryption  table  for

protection . Before you use this function , you

have to set as	below by function 7:

*** ENCRYPTION	TABLE ENABLE ***

*** FUNCTION D : DISPLAY & EDIT

To  display  and edit the content of the memory

buffer	 directly.  DEBUG.COM. is not neccessary

for  this  function.  Press Q to return to main

menu.

*** FUNCTION Q : QUIT

To return to DOS and save the current status of

the chips (MFG. TYP. ... etc.)	into the 51P.DAT

.  Next  time	when  you execute 51P02.EXE, the

same	IC   TYPE  as  the  last  time	will  be

automatically set .

�						       page 6-1

CHAPTER	V   MICROCONTROLLER ( 8748 Series , Z8 Series .. )

========================= FUNCTION DESCRIPTION =================

** Files required :

1. filename02.EXE : the	main program to	be executed.

2. filename.DAT	  : to	save  the  previous running data

after function Q was executed.

3. SETUP02.EXE

SETUP.DAT

(This file is for I/O address setting)

REMARK  : Please place above files	in the same working disk

drive.

***  The  function  menu  will be automatically	displayed on the

screen  as	 showed	 below after filename02.EXE (like 48P02.

EXE ) is executed.

8748 PROGRAMMER   V3.0      *MFG.:INTEL *ZIP:1

*TYP.:8748� *PROG.:intellegent

MAIN MANU 		   *VPP.:21.0V *VCC.:5V

==================================

1. DIR

2. LOAD	OBJ FILE TO MEMORY BUFFER

3. SAVE	MEMORY BUFFER TO DISK

4. DEBUG MEMORY	BUFFER

5. GANG	SIZE

6. PROGRAMMING ALGORITHM

M. MANUFACTURER

T. TYPE

B. BLANK CHECK

P. PROGRAM	 A. AUTO

R. READ		 V. VERIFY

C. COMPARE	 D. DISPLAY & EDIT

S. SECURITY FUSE BLOW

Q. QUIT

SELLECT	WHICH NUMBER ?

REMAKE 1 : Above right angle

* MFG.  stands for manufacturer(refer to	function

M)

* TYP.  stands for TYPE (refer to function T)

* VPP.  stands  for  the	 programming  voltage. (

refer to	function T)

* ZIP.  stands  for  the	number of SOCKET.( refer

to function 5 )

* PROG. stands  for  the	 programming algorithm &

time (refer to function 6)

* VCC.  stands for VCC voltage in programming.

�                                                       page 6-2

REMAKE 2 : Above	data  about  MFG.  TYP.	... etc. will be

saved	into filename.DAT file as the function Q

is  executed  .  The  data  will  be  read from

filename.DAT file as filename02.EXE is	executed

next  time  and  be  automatically set	the same

status	as the last time.

REMAKE 3 : After	filename02.EXE	is  executed,the PC will

open  a  64K  memory space as MEMORY BUFFER for

user  .  The  address	for MEMORY BUFFER is set

from  0000H--FFFF  (64K)  .  Data read	from the

EPROM	or  the	 DISK  will  be	 saved in MEMORY

BUFFER.

*** FUNCTION 1 : DIR

To list the files directory of	the DISK.

(under	the same DOS system)

FORMAT : [d:] [path] [filename {.ext}] [/p] [/w]

<ESC> back to main menu.

REMAKE : COMMAND.COM  must be put in drive (A:)	so as to

execute DOS COMMAND.

*** FUNCTION 2 : LOAD OBJ FILE INTO MEMORY BUFFER

To load .OBJ file from	DISK into MEMORY BUFFER.

You  may specify the path, .OBJ file file name,

the  buffer  starting	address	 to be loaded or

press <ESC> to	return to main menu.

FORMAT : Enter file name to be loaded:

[d:][path][filename .OBJ]

Enter buffer starting address:

[n]  ----  Usually  '0'  is  input  and started

from  the  first  address  in MEMORY

BUFFER.

or press <ESC>	to return to main menu.

REMARK : Please	load the OBJ CODE file.	If the .HEX file

was loaded, please use	the HEXOBJ02.EXE file to

convert  it  into  .OBJ  file	before you input

data.

*** FUNCTION 3 : SAVE MEMORY BUFFER TO DISK

To  save  data	from MEMORY BUFFER into	the disk

file  .  You  may  specify  the  path,	the .OBJ

filename,  the	 buffer	starting and end address

to  be	 saved	or press <ESC> to return to main

menu.

�                                                       page 6-3

FORMAT : Enter file name to be saved:

[d:][path][filename.OBJ]

Enter buffer starting address :

[n1] ---- the starting	address

Enter buffer end address :

[n2] ---- the end address

or press <ESC>	to return to main menu

REMARK : Above data will be saved in OBJ CODE format.

*** FUNCTION 4 : DEBUG MEMROY BUFFER

To debug and modify your memory buffer.

REMARK : Please	 place	DEBURG.COM  in	the working disk

drive and press any key to start (see DEBURG in

DOS). The display on the screen show :

Memory	buffer starting	address	at 4356:0000

|

|__ data

segment

<First	enter RDS to change data segment>

--RDS

--DS:1560

--4356	 ---- input data segment of memory buffer

start address

*** FUNCTION 5 : GANG SIZE

To  select  the  number  of SOCKET in a working

batch ( 1-4 SOCKET(S)).

REMAKE : If  you  want	to  expand  it to 4 SOCKETS, you

shall be asked	to buy additional 4 ZIP	sockets.

*** FUNCTION 6 : PROGRAMMING ALGORITHM

To select the type of the programming algorithm.

REMAKE : The   programming  algorithm  is  automatically

selected  when	 you change the	MFG. or	TYP. but

you  can change it by yourself	only you make it

in accordance with the	IC programming rules.

*** FUNCTION M : MANUFACTURER

To select the manufacturer of the chips.

*  Please select	the correct manufacturer because

the  programming algorithm of each manufacturer

is quite different.

�                                                       page 6-4

*  If  IC's  manufacturer  is  not  found, please

select	the first set "DON'T CARE".

*  If DON'T CARE is selected, you must assure the

TYPE	and  the  programming  voltage.	 If  the

programming  voltage  is unknown, please select

the  low  voltage  to test. If	failed,	try high

voltage.

*** FUNCTION T : TYPE

Use keys <1><2>...<A><B>...etc, to select TYPE.

The  correct  programming voltage and algorithm

will be automatically set as you select TYPE .

CAUTION : Please	 select	 the exactly correct programming

voltage and algorithm.

*** FUNCTION B : BLANK CHECK

To  check  whether  the chips are blank or not.

The length is decided by the section address. (

the  section  address	described  as below). If

there	 is  data  in  the  first  address,  the

displayed will	show error.

For example:

NO. 1    ERROR   AT 138

(The first EPROM at 0138 is not blank)

<Y> : start to check

<C> : to change the address

<ESC> : back to main menu

Section address : CHIP STARTING ADR.	:  the	starting address

of  the  chip  on the

socket

CHIP      END ADR.	:  the	end  address  of

the	 chip	on   the

socket

BUFFER STARTING ADR.: the	starting address

of memory buffer

BUFFER  CHECK  SUM	:  accumulated	by  BYTE

value.

< Above is presented by ASC	II >

BUFFER  CHECK  SUM's expression is accounted from the BUFFER

STARTING  ADDRESS,	its  length  =	(CHIP END ADR )	- ( CHIP

STARTING ADR.)

REMARK : Sometimes	a  damaged  chip  will be mistaken for a

blank chip	by using BLANK CHECK.

�                                                       page 6-5

*** FUNCTION P : PROGRAM

To  program  the chip with data from the memory

buffer	 to  EPROM. It will VERIFY automatically

after finishing program and display the result.

The  address  and  the	 length	 are  decided by

section address.

<Y> : start to program

<C> : to change the address

<ESC> : back to main menu

*** FUNCTION A : AUTO

To program automatically.

** steps executed	automatically :

1. BLANK CHECK

2. PROGRAM

3. VERIFY

4. SECURITY FUSE BLOW

*** FUNCTION R : READ

To read the data on the chip into memory buffer.

<Y> : start to reat

<C> : to change the address

<ESC> : back to main menu

REMAKE : After completing reading data from EPROM to the

MEMORY	BUFFER,	first you have to assure whether

the  CHECK  SUM is correct or not. If it is not

correct,  it means that the data read is wrong.

If  the original CHECK	SUM in EPROM is	unknown,

please	 check whether the content of the MEMORY

BUFFER	 is correct or not by using the	function

D.

*** Function V : VERIFY

To  verify  the  data in the chips with that in

the memory buffer.

<Y> : start to verify

<C> : to change the address

<ESC> : back to main menu.

*** FUNCTION C : COMPARE

To  compare  the  data	in the chip with that in

the  memory buffer. The difference is displayed

in the	format:

�                                                       page 6-6

CHIP	ADDR : CHIP DATA - ( BUFFER ADDR : BUFFER DATA )

As  the  difference is	displayed, you may press

<CTRL-S>  to  hold the	display	or press any key

to  continue  or  press <ESC> to return to main

menu.

<Y> : start to compare

<C> : to change the address

<ESC> : back to main menu

*** FUNCTION S : SECURITY BIT PROGRAMMING

To  program  security bit to protect inside ROM

from being read . If the U.V. eraser is used to

move  the  protection of the Security bit , the

inside	ROM programs will be erased , too.

*** FUNCTION D : DISPLAY & EDIT

To  display  and edit the content of the memory

buffer	 directly.  DEBUG.COM. is not neccessary

for  this  function.  Press Q to return to main

menu.

*** FUNCTION Q : QUIT

To return to DOS and save the current status of

the chips (MFG. TYP. ..etc.) into the filename.

DAT . Next time when you execute filename02.EXE

, the  same  IC TYPE  as  the last time will be

automatically set .

�						       page 7-1

CHAPTER	VII    Bipolar PROM

===================== FUNCTION DESCRIPTION ====================

** Files required :

1. BPP02.EXE : the main	program	to be executed.

2. BPP.DAT   : to  save	 the previous running data after

function	Q was executed.

3. SETUP02.EXE

SETUP.DAT

(This file is for I/O address setting)

REMARK  : Please place above files	in the same working disk

drive.

***  The  function  menu  will be automatically	displayed on the

screen as showed below after BPP02.EXE is executed.

BPROM PROGRAMMER 	  V3.0 	    * MFG.: AMD

* TYP.: 27S20-255*4

MAIN  MENU

===============================

1. DIR

2. LOAD OBJ FILE TO MEMORY BUFFER

3. SAVE MEMORY BUFFER TO DISK

4. DEBUG MEMORY BUFFER

5. SWAP BUFFER DATA

M. MANUFACTURER

T. TYPE

B. BLANK CHECK

P. PROGRAM	    A. AUTO

R. READ	    V. VERIFY

C. COMPARE	    D. DISPLAY & EDIT

Q. QUIT

SELECT WHICH NUMBER?

REMARK 1 : above right angle

* MFG. stands for manufacturer (see function M)

* TYP. stands for type (see function T)

REMARK 2 : Above	data  about  MFG.  TYP.	... etc. will be

saved	into  BPP.DAT  file as the function Q is

executed.  The	 same  data  will  be  read from

BPP.DAT file as BPP02.EXE is executed next time

and be	automatically set the same status as the

last time.

REMARK 3 : After BPP02.EXE is executed, the PC will open a

64K memory space as MEMORY BUFFER for user. The

address  for MEMORY BUFFER is set from	0000H --

�                                                       page 7-2

FFFF  (64K).  Data  read  from	the EPROM or the

DISK will be saved in MEMORY BUFFER.

*** FUNCTION 1 : DIR

To list the files directory of	the disk.

( under the same DOS system )

FORMAT : [d:][path][filename {.ext}][/p][/w]

or <ESC> back to main menu

REMARK :  COMMAND.COM  must be put in drive A  so as to

execute DOS COMMAND.

*** FUNCTION 2 : LOAD OBJ FILE INTO MEMORY BUFFER

To load .OBJ file from	DISK into MEMORY BUFFER.

You  may specify the path, .OBJ file file name,

the  buffer  starting	address	 to be loaded or

press <ESC> to	return to main menu.

FORMAT : Enter file name to be loaded:

[d:][path][filename .OBJ]

Enter buffer starting address:

[n] ---- Usually '0' is input and started from

the first address in MEMORY BUFFER.

or press <ESC>	to return to main menu.

REMARK : Please	load the OBJ CODE file.	If the .HEX file

was loaded, please use	the HEXOBJ02.EXE file to

convert  it  into  .OBJ  file	before you input

data.

*** FUNCTRION 3	: SAVE MEMORY BUFFER TO	DISK

To save data from MEMORY BUFFER into the disk .

You  may  specify  the	path, the .OBJ filename,

the buffer starting and end address to	be saved

or press <ESC>	to return to main menu.

FORMAT	: Enter	file name to be	saved:

[d:][path][filename.OBJ]

Enter	buffer starting	address	:

[n1] ---- the	starting address

Enter	buffer end address :

[n2] ---- the	end address

or press <ESC> to return to main menu

REMARK	: Above	data will be saved in OBJ CODE format.

*** FUNCTION 4 : DEBUG MEMROY BUFFER

To debug and modify your memory buffer.

�                                                       page 7-3

REMARK : Please	 place	DEBUG.COM  in	the working disk

drive and press any ket to start ( see DEBUG in

DOS). The display screen looks	like this:

Memory	buffer starting	address	at 4356:0000

|

|__ data

segment

<First	enter RDS to change data segment>

--RDS

--DS:1560

--4356	 ---- input data segment of memory buffer

start address

*** FUNCTION 5 : SWAP BUFFER DATA

To  exchange  data in HIGH NIBBLES with that in

LOW NIBBLES in	the memory buffer. The length is

decided by section address.

For example : the orinigal memory buffer is	   0010: C3

after this function is	executed   0010: 3C

*** FUNCTION M : MANUFACTURER

To select the manufacturer of the chips.

* Please	 select	the correct manufacturer because

the  programming algorithm of each manufacturer

is quite different.

*** FUNCTION T : TYPE

Use keys <1><2>...<A><B>...etc, to select TYPE.

CAUTION : Please	 select	 the exactly correct programming

voltage and algorithm.

*** FUNCTION B : BLANK CHECK

To  check  whether  the chips are blank or not.

The length is decided by the section address. (

the  section  address	described  as below). If

there	is  data  inside  the address, the error

will be displayed.

For example:

NO. 1    ERROR   AT 138

(BPROM	at 0138	is not blank)

<Y> : start to check

�                                                       page 7-4

<C> : to change the address

<ESC> : back to main menu

Section address : CHIP STARTING ADR.	:  the	starting address

of  the  chip  on the

socket

CHIP      END ADR.	:  the	end  address  of

the	 chip	on   the

socket

BUFFER STARTING ADR.: the	starting address

of memory buffer

BUFFER  CHECK  SUM	 : accumulated	by  BYTE

value.

< Above is presented by ASC	II >

BUFFER CHECK SUM's expression is accounted from the BUFFER

STARTING ADDRESS, its length = (CHIP END ADR ) - ( CHIP

STARTING ADR.)

REMARK : Sometimes	a  damaged  chip  will be mistaken for a

blank chip	by using BLANK CHECK.

*** FUNCTION P : PROGRAM

To  program  the chip with data from the memory

buffer	 to  EPROM. It will VERIFY automatically

after finishing program and display the result.

The  address  and  the	 length	 are  decided by

section address.

<Y> : start to program

<C> : to change the address

<ESC> : back to main menu

*** FUNCTION A : AUTO

To program automatically.

** steps	executed automatically :

1. BLANK	CHECK

2. PROGRAM

3. VERIFY

*** FUNCTION R : READ

To read the data on the chip into memory buffer

to be saved.

<Y> : start to reat

<C> : to change the address

<ESC> : back to main menu

REMARK : If  there  is	only  4	Bit data from BPROM, the

�                                                       page 7-5

data  will  be	 read into the 4-bit only in low

nibbles  of  the  memory buffer as the	function

READ  is executed. It won't affect the 4 bit in

high nibbles.

REMARK : After	completing  reading  data  from	BPROM to

MEMORY	BUFFER,	first you have to assure whether

the  CHECK  SUM is correct or not. If it is not

correct,  it means that the data read is wrong.

If  the original CHECK	SUM in BPROM is	unknown,

please	 check	whether	 the  content  of MEMORY

BUFFER	 is correct or not by using the	function

D.

*** FUNCTION V : VERIFY

To  verify  the  data in the chips with that in

memory	buffer.

<Y> : start to verify

<C> : change	the address

<ESC> : back to main menu

REMARK : If  there is only 4 bit data from BPROM, as the

function  VERIFY  is executed the computer will

verify	 with the 4 bit	data only in low nibbles

of the	memory buffer.

*** FUNCTION C : COMPARE

To  compare  the  data	in the chip with that in

the  memory buffer. The difference is displayed

in the	format:

CHIP ADDR :	CHIP DATA - ( BUFFER ADDR : BUFFER DATA)

As  the  difference is	displayed, you may press

<CTRL-S>  to  hold the	display	or press any key

to  continue  or  press <ESC> to return to main

menu.

<Y> : start to compare

<C> : to change the address

<ESC> : back to main menu

REMARK : If  there is only 4 bit data from BPROM, as the

function  COMPARE is executed the computer will

compare with the 4 bit	data only in low nibbles

of the	memory buffer.

*** FUNCTION D : DISPLAY & EDIT

To  display  and edit the content of the memory

�                                                       page 7-6

buffer	 directly.  DEBUG.COM. is not neccessary

for  this  function.  Press Q to return to main

menu.

*** FUNCTION Q : QUIT

To return to DOS and save the current status of

the chips (MFG. TYP. ... etc.)	into the BPP.DAT

file.	Next time as you execute BPP02 .EXE, the

same IC TYPE will be automatically selected.

�						       page 8-1

CHAPTER	VIII	PAL, FPL, GAL, PEEL, EPLD

=======================	FUNCTION DESCRIPTION ====================

**** NOTES *****

1.  To use the chips such	as PAL / FPL / GAL / PEEL / EPLD

etc.,	you  have to select the	correct	manufacturer and

the  TYPE fisrt before	you read / compare / program. It

is  possible  to  demage IC even in READ MODE or VERIFY

MODE if the wrong manufacturer	and TYPE are selected.

2.  If you want to bulk program PAL, please try one or two

first.	 Should	there be no problem in actural practice,

you  can start	to bulk	program	PAL. Unneccessary damage

can be	avoided	due to error operation by following this

instruction.

** File	required :

1. (filename.EXE) : the	main program to	be executed such

as GAL02.EXE

2. PALFORM\fusemap.FRM :

the	subdirectory including the blank

form   files   of	all   PAL  chips

available for the package.

3. (filename.DAT) : to	save  the  previous running data

while   the	 the  function	QUIT  is

executed, such as GAL.DAT

4. SETUP02.EXE

SETUP.DAT

(This file is for I/O address setting)

REMARK  : Please place above files	in the same working disk

drive.

***  The  function  menu  will be automatically	displayed on the

screen   as   showed  below  after	 filename.EXE  (such  as

GAL02.EXE)	is executed.

�                                                       page 8-2

GAL PROGRAMMER   V3.00      *MFG.: LATTICE	      CHECK SUM

*TYPE: NONE-TYPE		 =:0

MAIN MENU	       *FUSE MAP: NONE-MAP

===========================

1. DIR

2. LOAD	FUSE MAP FROM DISK

3. SAVE	FUSE MAP TO DISK

4. EDIT	FUSE MAP

M. MANUFACTURER

T. TYPE

B. BLANK CHECK

P. PROGRAM	  A. AUTO

R. READ		  V. VERIFY

E. BULK	ERASE

S. SECURITY FUSE BLOW

Q. QUIT

REMARK 1 : above right angle

* MFG. stands for manufacturer (see function M)

* TYP. stands for TYPE (see function T)

* FUSE MAP. stands  for	the filename loaded into

FUSE	BUFFER

* CHECK SUM. stands for the CHECK SUM of	the FUSE

BUFFER

REMARK 2 : Above	running	 data  such as MFG. TYP. ...etc.

will  be  saved into filename.DAT and be loaded

from  filename	 .DAT next time	when you execute

filename.EXE.	The  data  will	be automatically

selected the same as the last time.

REMARK 3 : The  PC will open a memory space as FUSE BUFFER

after executing filename.EXE.

*** FUNCTION 1 : DIR

To list the files directory from the disk.

(under	the same DOS system)

FORMAT : [d:][path][filename{.rxt}][/p][/w]

or <ESC> bact to main menu

REMARK :  COMMAND.COM  must be put in driver A	so as to

execute DOS COMMAND,

*** FUNCTION 2 : LOAD FUSE MAP FROM DISK

To  load  the	JEDEC  FORMAT file from	the disk

into  FUSE  BUFFER, enter the path, followed by

the  file  name  for FUSE MAP or press	<ESC> to

return	to main	menu.

�                                                       page 8-3

FORMAT : [d:][path][filename.jed]

<ESC> back to main menu

REMARK : Please	 use the standard format for JEDEC FILE,

such  as  the	JEDEC  FORMAT assembled	by ABEL,

PALASM, AMAZE,	CUPL, PLAN... etc.

*** FUNCTION 3 : SAVE FUSE MAP TO DISK

To  save  fuse	 map  in  JEDEC	FORMAT from FUSE

BUFFER	 to  the  disk,	 enter	the path and the

JEDEC	FORMAT filename	or press <ESC> to return

to main menu.

FORMAT : [d:][path][filename.jed]

<ESC> back to main menu

*** FUNCTION 4 : EDIT FUSE MAP

To display or edit the	FUSE MAP.

<ESC> back to main mene.

To move the	cursor : <UP><DN><LF><RT><HOME><END>

To change the page : <PGUP><PGDN>

To enter the data  : <0><1><F>

_________________________________________________

|	     |	GAL. FPL. PEEL(18CV8) |	PAL, 22V10	|

|______|________________________|_________________|

|	  0  |	fuse blown	      |	 fuse not blown	|

|______|________________________|_________________|

|	  1  |	fuse not blown	      |	 fuse blown	|

|______|________________________|_________________|

* key F : set  the  whole  EDIT FUSE MAP	(FUSE BUFFER) as

BLANK

* (N) in	EDIT MAP stands	for no program function.

REMARK : This is a slow	way to edit FUSE MAP and easy to

make mistake, so it is	better for you to try to

use assembler software	to edit	FUSE MAP such as

ABEL, PALASM, AMAZE, CUPL PLAN	... etc.

(Please  refer	 to  the user manual of	kinds of

assembler software.)

*** FUNCTION M : MANUFACTURER

To select the manufacturer for	the chips.

For example: GAL02.EXE

�                                                       page 8-4

_______ MANUFACTURER ___________

| 1. LATTICE		   |

| 2. NS			   |

| 3. SGS			   |

| 4. VLSI			   |

| <ESC> back to main menu	   |

| SELECT NUMBER ?		   |

|______________________________|

Please	select the correct manufacturer.

REMARK : The  programming algorithm of each manufacturer

is quite different.

*** FUNCTION T : TYPE

To select TYPE.

For example : there are two types for GAL02.EXE TYPE

1. 20 PIN

such as GAL16V0-15/25/35

2. 24 PIN

such as GAL20V8-15/25/35

* Press <PGUP><PGDN> to select the chip of 20 pin

or 24 pin.

* Press <1><2>...<A><B>... etc. to select TYPE.

*** FUNCTION B : BLANK CHECK

To check whether the chips are	blank or not. If

there is data inside the chip,	the display will

show 'error'.

REMARK : The  PAL  which SECURITY FUSE is blown	may show

blank in doing	BLANK CHECK.

*** FUNCTION P : PROGRAM

To program the	chips.

__ PROGRAM ____________________

| Blank check or	not (Y/N/ESC)  |

|			       |

|_______________________________|

<Y> : BLANK	CHECK.	If the chip is blank, it will be

programmed. If	the chip is not	blank, the error

will  be  displayed  and  the	chip will not be

programmed.

<N> : No BLANK CHECK. Program directly.

<ESC> : back to main menu

*  If  as  you press key	<Y> , the BLANK	CHECK is

�                                                       page 8-5

O.K.  but  errors  occur in programming, the IC

may be	damaged.

*  Although  the	PAL which SECURITY FUSE	is blown

is  checked  blank,  it is not	programmable any

more.

* If PEEL (18CV8	,22CV10) and GAL (16V8,	20v8) of

EEPROM	 TYPE  are  not	 blank,	 please	 use the

function  E  :	 BULK ERASE to clear the content

before	you program them.

*  Please  use ERASER to	clear first the	chips of

EPROM TYPE such as CPAL.

*** FUNCTION A : AUTO

** steps executed automatically

1. BLANK CHECK

2. PROGRAM

3. VERIFY

4. SECURITY FUSE BLOW

(  The	 error will be showed if anyone	of above

can't be executed)

*** FUNCTION R : READ

To read the data in the chips.	The CHECK SUM is

on the top right angle of the screen .

REMARK : You can check the data	loaded from the	chips by

using the function 4.

*** FUNCTION V : VERIFY

To  verify  the  data in the chips with that in

the FUSE BUFFER.

*** FUNCTION S : SECURITY FUSE BLOW

To protect the chips from being read or writted.

*** FUNCTION Q : QUIT

To  return  to	DOS and	to save	the running data

(MFG. TYP. JEDEC FILE)	into filename.DAT  (such

as GAL.DAT, PAP.DAT etc.)

*** EEPROM TYP such as

PEEL (18CV8	, 22CV10) & GAL	(16V8 ,	20V8) special function ***

�                                                       page 8-6

*** FUNCTION E : BULK ERASE

To clear data in the chips and	do BLANK CHECK.

REMARK : After ERASE, the protection is	also erased.

�						       page 9-1

CHAPTER	IX  : DIGITAL IC & MEMORY TESTER

====================== FUNCTION	DESCRIPTION ====================

** Files required :

1. DMT02.EXE : the main	program	to be executed.

2. DMT.DAT   : to  save	 the previous running data after

function	Q is executed.

3. SETUP02.EXE

SET.DAT

(This file is for I/O address setting)

REMARK  : Please place above files	in the same working disk

drive.

***  The  function  menu  will be automatically	displayed on the

screen as showed below after DMT02.EXE is executed.

DIGITAL IC & MEMORY TESTER    V3.0

MAIN MENU

=============================================

1. DIR

2. LOAD	TEST PATTERN & TESTING

3. SAVE	TEST PATTERN

4. EDIT	TEST PATTERN

5. DEBUG TEST PATTERN

T. TTL TESTER

C. CMOS	TESTER

M. MEMORY TESTER

Q. QUIT

SELECT WHICH NUMBER?

*** FUNCTION 1 : DIR

To list the files directory from the disk.

(under	the same DOS system)

FORMAT : [d:][path][filename{,ext}][/p][/w]

<ESC> back to main menu

REMARK : COMMAND.COM  must  be place into the drive A so

as to execute DOS COMMAND.

*** FUNCTION 2 : LOAD TEST PATTERN & TESTING

To  load  the	TEST  PATTERN file from	the disk

into  the  memory  buffer. (Please refer to the

format	in function 4) Enter the path, file name

and  press  any  key  to test or press	<ESC> to

return	to main	menu.

�                                                       page 9-2

FORMAT : [d:][path][filename]

<ESC> back to main menu

*** FUNCTION 3 : SAVE TEST PATTERN

To load the TEST PATTERN from the memory buffer

into  the  disk to be filed. Enter the	path and

the  filename	or press <ESC> to return to main

menu.

FORMAT : [d:][path][filename.jed]

<ESC> back to main menu

REMARK : Above is saved	in BINARY CODE format.

*** FUNCTION 4 : EDIT TEST PATTERN

To edit the TEST PATTERN.

Please	 place	DEBUG.COM  into	the working disk

drive	before you press this function key, then

press	any  key to start. (Please see DEBUG. in

MS DOS)

<Q> back to main menu

REMARK : see examples

*** FUNCTION 5 : DEBUG TEST PATTERN

To   debug   and   display   the  test	 pattern

established by	<function 4> and test by step.

REMARK : see examples

*** FUNCTION T : TTL TESTER

To test the chips of TTL series.

<C> : change	number ( change	the tested IC number)

<A> : auto  search  IC  number ( search the IC number

ready for test	automatically and test it)

<SPACE> : testing (test selected	IC)

<ESC> : return	to main	menu.

REMARK 1 : change	number : simply	enter the number such as

138,  00,  07..etc.  (	 at most

three digitals)

REMARK 2 : auto search IC	number : will  or  won't display

the  IC number	with the

same function.

�                                                       page 9-3

EXAMPLE 1 : Result	: 04, 14, 19, 4069, 4584

**** END

EXAMPLE 2 : Result:

**** END

REMARK 3 : testing : test	and display GOOD or ERROR

*** FUNCTION C : CMOS TESTER

To test the chips of CMOS seires.

(please refer to <function T>)

*** FUNCTION M : MEMORY	TESTER

To test memory	.

1. 4164 - 64K*1	2. 4256	- 256K*1

3. 2114 - 1K*4	4. 6116	- 2K*8

5. 6264 - 8K*K	6. 6256	- 32K*8

(There are six sets to select in MEMORY)

<C> : change	number (to select MEMORY TYPE)

<SPACE> : testing

<ESC> :  back to main menu

*** FUNCTION Q : QUIT

to return to DOS

*** EXAMPLE :

The  pin  assignment of the testing socket is showed as

below:	( only 24 pin used, 3 bi-direction I/O PORTs)

|-------v-------|

PC4 --|1	       24|-- PC7 or VCC

PC5 --|2	       23|-- PC6 or VCC

PC0 --|3	       22|-- PC3 or VCC

PC1 --|4	       21|-- PC2 or VCC

PA0 --|5	       20|-- PB7 or VCC

PA1 --|6	       19|-- PB6 or VCC

PA2 --|7	       18|-- PB5

PA3 --|8	       17|-- PB4

PA4 --|9	       16|-- PB3

PA5 --|10	       15|-- PB2

PA6 --|11	       14|-- PB1

GND	or PA7 --|12	       13|-- PB0

|---------------|

24 PIN SOCKET

�                                                       page 9-4

There	are  3 bi-direction I/O	PORTs (PA, PB and PC) to

connect with socket.

The procedures	to establish your own TEST PATTERN is :

1. select VCC and GND type code .(VCC)

2. set	direction (DIRA, DIRB & DIRC) or PA, PB	and PC.

3. establish test pattern for PORT A .(TPA)

4. establish test pattern for PORT B .(TPB)

5. establish test pattern for PORT C .(TPC)

6. total sets of test pattern.	(NO.)

So  you  have	to establish the following data	on paper

first.

(1).	VCC (VCC & GND code)  =

(2).	DIRA (direction	of PA) =

(3).	DIRB (direction	of PB) =

(4).	DIRC (direction	of PC) =

(5).	TPA (test pattern of PA) =

(6).	TPB (test pattern of PB) =

(7).	TPC (test pattern of PC) =

(8).	NO. (total sets	of test	pattern) =

Here  are two examples	to establish the test pattern on

paper.

Expamle 1 : to establish TEST PATTERN for 74LS00

|-------v-------|

PC4 --|1	       24|-- PC7

PC5 --|2	       23|-- PC6

PC0 --|3	       22|-- PC3

PC1 --|4	       21|-- PC2

PA0 --|5____________20|-- PB7

INPUT  PA1 --|6(1A)	  (VCC)19|-- PB6

INPUT  PA2 --|7(1B)	   (4B)18|-- PB5  INPUT

OUTPUT  PA3 --|8(1Y)	   (4A)17|-- PB4  INPUT

INPUT  PA4 --|9(2A)	   (4Y)16|-- PB3  OUTPUT

OUTPUT  PA5 --|10(2B)   (3B)15|-- PB2  INPUT

INPUT  PA6 --|11(2Y)   (3A)14|-- PB1  INPUT

GND --|12(GND)  (3Y)13|-- PB0  OUTPUT

|---------------|

PIN ASSIGNMENT OF 74LS00

PROCEDURES:

(1).	VCC and	GND type code :	00

The VCC	& GND code are defined as below	:

�                                                       page 9-5

___________________________________

|				    |

|  14	pin chip ---  code is :	00  |

|  16	pin chip ---  code is :	01  |

|  18	pin chip ---  code is :	02  |

|  20	pin chip ---  code is :	03  |

|  22	pin chip ---  code is :	04  |

|  24	pin chip ---  code is :	05  |

|_________________________________|

74LS00 is	a 14 pin chip, so VCC code is  00

(2) set DIRA, DIRB or PA and	PB.(don't care PC)

The	code bit of direction is defined as below :

________________________________________

|					   |

|		VCC  pin code  bit is :	0  |

|		GND  pin code  bit is :	0  |

|	      INPUT  pin code  bit is :	1  |

|	     OUTPUT  pin code  bit is :	0  |

|	 DON'T CARE  pin code  bit is : 1  |

|______________________________________|

*** Set DIRA of PA ***

PORT A :  PA7	 PA6  PA5  PA4	PA3  PA2  PA1  PA0

GND	 OUT  IN   IN	OUT  IN	  IN	X

0	  0    1    1	 0    1	   1	1 = 37

so DIRA = 37

*** Set DIRA of PB ***

PORT B :  PB7	 PB6  PB5  PB4	PB3  PB2  PB1  PB0

X	 VCC  IN   IN  OUT   IN	  IN   OUT

1	  0    1    1	 0    1	   1	0 = B6

so DIRB = B6

(3). establish TEST PATTERN

level define :  0 = low level

1 = high	level

Test pattern of PA :

PORT A :  PA7  PA6	PA5  PA4  PA3  PA2  PA1	 PA0

GND  OUT	 IN   IN  OUT	IN   IN	  X

..........................................................

(1)  test bit input		  0    0	0    0

expected output	    1		   1

--------------------------------------

conbination       0    1	  0    0   1	0    0	  0 = 48

�                                                       page 9-6

..........................................................

(2)  test bit input		  0    1	0    1

expected output	    1		   1

--------------------------------------

conbination       0    1	  0    1   1	0    1	  0 = 5A

..........................................................

(3)  test bit input		  1    0	1    0

expected output	    1		   1

--------------------------------------

conbination       0    1	  1    0   1	1    0	  0 = 6C

..........................................................

(4)  test bit input		  1    1	1    1

expected output	    0		   0

--------------------------------------

conbination       0    0	  1    1   0	1    1	  0 = 36

..........................................................

so TPA	is : 48	5A 6C 36 ( total 4 sets)

PORT B :  PB7  PB6	PB5  PB4  PB3  PB2  PB1	 PB0

X   VCC	IN   IN	  OUT  IN   IN	 OUT

..........................................................

(1)  test bit input		 0    0		0    0

expected output			   1		  1

--------------------------------------

conbination      0    1	 0    0	   1	0    0	  1 = 49

..........................................................

(2)  test bit input		 0    1		0    1

expected output			   1		  1

--------------------------------------

conbination       0    1	 0    1	   1	0    1	  1 = 5B

..........................................................

(3)  test bit input		 1    0		1    0

expected output			   1		  1

--------------------------------------

conbination       0    1	 1    0	   1	1    0	  1 = 6D

...........................................................

(4)  test bit input		 1    1		1    1

expected output			   0		  0

--------------------------------------

conbination       0    1	 1    1	   0	1    1	  0 = 76

...........................................................

so TPB is : 49 5B 6D 76  ( total 4 sets)

(4). The total sets of test pattern is 4.

Now,  we have established all the code	and patterns, as

shown below :

1. VCC	(VCC & GND code)  = 00

2. DIRA (direction of PA) = 37

3. DIRB (direction of PB) = B6

4. DIRC (direction of PC) = DON'T CARE

5. TPA	(test pattern of PA) = 48 5A 6C	36

�                                                       page 9-7

6. TPB	(test pattern of PB) = 49 5B 6D	76

7. TPC	(test pattern of PC) = DON'T CARE

8. NO.	(total sets of test pattern) = 4

(5). EDIT TEST	PATTERN

1. Run	 DMT02.EXE  file,  it  will  open  a  BUFFER for

editing  user's  test  pattern  and  the  BUFFER  is

assigned as	below :

buffer  0  to 7F used for TPA

buffer  80 to FF used for TPB

buffer 100 to 17F used for TPC

buffer 180 used for	DIRA

buffer 181 used for	DIRB

buffer 182 used for	DIRC

buffer 183 used for	VCC

buffer 184 used for	NO.

2. Key	 in  4	to  do the edit	function, and the screen

will shown as,

Memory  buffer starting address at 2133:0000 (buffer

starting address)

-RDS     <-- enter RDS

DS 2C2A

:2133    <-- enter buffer starting address

-E0

2133:0000  00-48  00-5A  00-6C 00-36 <--enter TPA

-E80

2133:0080  00-49  00-5b  00-6D 00-76 <--enter TPB

-E180

2133:0180  00-37	<--enter DIRA

-E181

2133:0181  00-B6	<--enter DIRB

-E183

2133:0183  00-00	<--14 pin code is 00

-E184

2133:0184  00-04	<--4 sets of test pattern

-Q			<--back	to main	menu

(6). DEBUG TEST PATTERN

Type 5 to	debug user's test pattern, as below

1. Total pattern sets : 04

2. VCC & GND	code : 00

3. port(A) in/out code : 37

4. port(B) in/out code : B6

�                                                       page 9-8

Put  IC  on socket, then press	any key	to test	by step,

or press <ESC>	to quit.

1. Pattern write   port(A)=48   port(B)=49

read   port(A)=48   port(B)=49

2. Pattern write   port(A)=5A   port(B)=5B

read   port(A)=5A   port(B)=5B

3. Pattern write   port(A)=6C   port(B)=6D

read   port(A)=6C   port(B)=6D

4. Pattern write   port(A)=36   port(B)=76

read   port(A)=36   port(B)=76

****** END.  Press any key to continue.

(7). SAVE TEST	PATTERN

Type 2 to	save user's test pattern.

Expamle 2 : to edit TEST PATTERN for 74LS244

|-------v-------|

PC4 --|1	       24|-- PC7

PC5 --|2____________23|-- PC6

INPUT  PC0 --|3(1G)	  (VCC)22|-- PC3

INPUT  PC1 --|4(1A1)  (2G) 21|-- PC2  INPUT

OUTPUT  PA0 --|5(2Y4)  (1Y1)20|-- PB7  OUTPUT

INPUT  PA1 --|6(1A2)  (2A4)19|-- PB6  INPUT

OUTPUT  PA2 --|7(2Y3)  (1Y2)18|-- PB5  OUTPUT

INPUT  PA3 --|8(1A3)  (2A3)17|-- PB4  INPUT

OUTPUT  PA4 --|9(2Y2)  (1Y3)16|-- PB3  OUTPUT

INPUT  PA5 --|10(1A4) (2A2)15|-- PB2  INPUT

OUTPUT  PA6 --|11(2Y1) (1Y4)14|-- PB1  OUTPUT

PA7 --|12(GND) (2A1)13|-- PB0  INPUT

|---------------|

PIN	ASSIGNMENT OF 74LS244

1. VCC	and GND	type code is 03

2, set	DIRA,  DIRB and	DIRC or	PORT A ,PORT B and PORT

C.

PORT A :  PA7  PA6	PA5  PA4  PA3  PA2  PA1	 PA0

GND  OUT	IN   OUT  IN   OUT  IN	 OUT

0    0	 1    0	   1	0    1	  0

so DIRA of PORT A is : 2A

PORT B :  PB7  PB6	 PB5  PB4  PB3	PB2  PB1  PB0

OUT  IN	 OUT  IN   OUT	 IN  OUT   IN

0    1	  0    1    0	 1    0	   1

�                                                       page 9-9

so DIRB of PORT B is : 55

PORT C :  PC7  PC6	 PC5  PC4  PC3	PC2  PC1  PC0

X    X	  X    X   VCC	 IN  IN	  IN

1    1	  1    1    0	 1    1	   1 = F7

so DIRC of PORT C is : F7

3. establish TEST PATTERN

PORT A :  PA7  PA6	PA5  PA4  PA3  PA2  PA1	 PA0

GND  OUT	IN   OUT  IN   OUT  IN	 OUT

(1)  test bit input		 1	   1	     1

expected output	    1	      1		1	  1

--------------------------------------

conbination       0    1	 1    1	   1	1    1	  1 = 7F

...........................................................

(2)  test bit input		 0	   0	     0

expected output	    1	      1		1	  1

--------------------------------------

conbination       0    1	 0    1	   0	1    0	  1 = 55

...........................................................

(3)  test bit input		 1	  1	     1

expected output	    1	      1		1	  1

--------------------------------------

conbinatio�       0    1	 1    1	   1	1    1	  1 = 7F

...........................................................

so TPA is : 7F 55 7F

PORT B :  PB7  PB6	PB5  PB4  PB3  PB2  PB1	 PB0

OUT  IN	OUT  IN	  OUT  IN   OUT	  IN

(1)  test bit input	    1	      1		1	  1

expected output   1	 1	   1	     1

--------------------------------------

conbination       1    1	 1    1	   1	1    1	  1 = FF

...........................................................

(2)  test bit input	    0	      0		0	  0

expected output   1	 1	   1	     1

--------------------------------------

conbination       1    0	 1    0	   1	0    1	  0 = AA

...........................................................

(3)  test bit input	    1	      1		1	  1

expected output   1	 1	   1	     1

--------------------------------------

conbination       1    1	 1    1	   1	1    1	  1 = FF

...........................................................

so TPB	is : FF	AA FF

PORT C :  PC7  PC6	PC5  PC4  PC3  PC2  PC1	 PC0

X    X	 X    X	  VCC  IN   IN	 IN

(1)  test bit input				     1	  0

expected output				0

�                                                       page 9-10

--------------------------------------

conbination       0    0	 0    0	   1	0    1	  0 = 0A

...........................................................

(2)  test bit input				     0	  1

expected output				1

--------------------------------------

conbinatio�       0    0	 0    0	   1	1    0	  1 = 0D

...........................................................

(3)  test bit input				     1	  1

expected output				1

--------------------------------------

conbination       0    0	 0    0	   1	1    1	  1 = 0F

..............................................................

so TPC is	: 0A,0D,0F

(4). Total sets of test pattern is 3

Now,  we have established all the code	and patterns, as

shown below :

1. VCC (VCC & GND code)  = 03

2. DIRA (direction of PA) = 2A

3. DIRB (direction of PB) = 55

4. DIRC (direction of PC) = F7

5. TPA (test	pattern	of PA) = 7F 55 7F

6. TPB (test	pattern	of PB) = FF AA FF

7. TPC (test	pattern	of PC) = 0A 0D 0F

8. NO. (total sets of test pattern) = 3

(5). EDIT TEST	PATTERN

1. Run  DMT-02.EXE  file , it  will open a BUFFER for

editing  user's  test  pattern  and  the BUFFER is

assigned as below	:

buffer  0	 to 7F used for	TPA

buffer  80 to FF used for	TPB

buffer 100 to 17F	used for TPC

buffer 180 used for DIRA

buffer 181 used for DIRB

buffer 182 used for DIRC

buffer 183 used for VCC

buffer 184 used for NO.

2. Key  in  4 to do the edit	function, and the screen

will shown as,

Memory   buffer	starting  address  at  2133:0000

(buffer starting address)

-RDS     <-- enter RDS

DS 2C2A

:2133    <-- enter buffer	starting address

�                                                       page 9-11

-E0

2133:0000	 00-7F	00-55  00-7F <--enter TPA

-E80

2133:0080	 00-FF	00-AA  00-FF <--enter TPB

-E100

2133:0100	 00-02	00-05  00-07 <--enter TPC

-E180

2133:0100	 00-2A	  <--enter DIRA

-E181

2133:0181	 00-55	  <--enter DIRB

-E182

2133:0182	 00-FF	  <--enter DIRC

-E183

2133:0183	 00-03	  <--20	pin code is 00

-E184

2133:0184	 00-03	  <--3 sets of test pattern

-Q		  <--back to main menu

(6). DEBUG TEST PATTERN

Type 5 to	debug user's test pattern, as below

1. Total pattern sets : 03

2. VCC & GND	code : 03

3. port(A) in/out code : 2A

4. port(B) in/out code : 55

5. port(C) in/out code : FF

Put  IC  on  socket, then	press any key to test by

step, or press <ESC> to quit.

1. Pattern write port(A)=7F port(B)=FF port(C)=02

read port(A)=7F port(B)=FF port(C)=02

2. Pattern write port(A)=55 port(B)=AA port(C)=05

read port(A)=55 port(B)=AA port(C)=05

3. Pattern write port(A)=7F port(B)=FF port(C)=07

read port(A)=7F port(B)=FF port(C)=07

****** END.  Press any key to continue.

(7). SAVE TEST	PATTERN

Type 2 to	save user's test pattern.

�						       page 10-1

CHAPTER	X   OTHER SOFTWARES DESCRIPTION

1. DASM48.EXE &	DASM51.EXE

DASM48.EXE  is  the	dis-assembler for 8748/49; DASM51.EXE is

the dis-assembler for 8751/52.

For example:

To execute DASM48.EXE

A>DASM48

HI-LO SYSTEM RESEARCH CO., LTD

8048/8049 DISASSEMBLER V1.0

-?

Command syntax

Dump memory    :	D[start	address[,end address]]

Enter	      :	E[start	address]

Disassembler   :	U[start	address[,end address]]

Define filename:	N[file spec.]

Load file      :	L[start	address]

Write file     :	W[start	address, end address]

Quit to DOS    :	Q

Query command  :	H or ?

NOTE: { ...} :	Contain	must be	specified

[ ...] :	Contain	optional

Address	limit :	2047 (7FF Hex)

comma(,) may be	replaced by blank, dot or TAB

The	usage  of each function	will be	displayed as soon as you

enter  ?.  The  usage  of those functions is	same to	DEBUG in

DOS.	Please refer to	the description	of DEBUG in DOS.

2. HEXOBJ.EXE & HEXOBJ2.EXE

HEXOBJ.EXE	is used to convert HEX	FORMAT into BINARY CODE.

HEXOBJ.EXE for 8 bit HEX file.

HEXOBJ2.EXE for 16 bit HEX file.

A> HEXOBJ

HEX TO OBJECT CONVERTER

HEX file name : test

OBJ file name : test

HEX file format {<I>intel <M>MOTOLORA <T> TEK }:I

Wait ...

INTEL 80 Hex to Binary converter

Convert complete

A>

�                                                       page 10-2

3. PARTS02.LST

This	 software is a record of all available programmable ICs.

Please print	PARTS02.LST to review if you don't know which IC

should be programmed	by which software.

PARTSO2.LST contains	five parts

a. EPROM / EEPROM

b. PAL /FPL /GAL / PEEL

C. Microcomputer

d. BPROM

e. IC & MEMORY TESTER

Each	part is	divided	as below:

Run file     : PAP02.EXE (V3.0)     ----- execute program

Manufacturer : MMI-A TYPE           ----- select MANUFACTURER

Type         :

10H8-2          12H6-2              ----- select TYPE

16C1-2          10L8-2

:               :

:               :

4. VERSION.DOC

the list for	all softwares upgraded.

5. MANUAL.DOC

the User' manual.

6. TEST02.EXE

the hardware	check.(	Please refter to CHAPTER XI 'DAMAGE

CHECK)

7. IOCHK02.EXE

the I/O ADDRESS check for universal programmer .

�                                                       page 11-1

CHAPTER	XI  DAMAGE CHECK

10.1 TEST02.EXE	 is mainly used	to test	whether	the programmer's

VL,  VM  &	VH voltage, ground, indicator light, and I/O BUS

are  normally performed. (Should there be any problem arise

during  working,  please contact the dealer or our	company.

We	 will fix and maintain your programmer at our soonest or

solve  problem  for  you.	Please	don't  try  to  fix your

programmer	by yourself.)

10.2 In	 testing, simply test the voltage and the status of each

pin on the	socket.

10.3 Please  prepare  the  following  testing  tool  required in

testing.

a.	SCOPE.

b.	DVM.

10.4 the  user testing procedure After executing TEST02.EXE, the

display shows as below:

-----   MAIN MENU	--------

User testing procedure:

(1) VL test and adjustment

(2) VM test and adjustment

(3) VH test and adjustment

(4) Ground	test

(5) I/O test

(6) Extra test

(Q) Quit

--------------------------

============ FUNCTION DESCRIPTION =============

The  +(-) 0.2 V difference	for the	following voltage values

is	allowed.

*** FUNCTION 1 : VL TESTING & ADJUSTMENT

To test and adjust VL voltage.	( VR1)

<S>: To  select  the  testing  voltage.  VL	 testing

voltage  is selected as 2.2V ,	4.5V, 5.9V 7.7V,

8.3V,	8.7V  etc.

--  totally six setting. In testing, you simply

test the voltage status at pin	40.

<P>: To  test each pin whether it's voltage value is

same to that as showed	on the screen. VL pin is

assigned at pin 25, 28, 29, 30, 32 34,	36 &40.

<ESC>: back to main menu.

�                                                       page 11-2

*** FUNCTION 2 : VM TESTING & ADJUSTMENT

To test and adjust VM voltage (VR3)

<S>: to  select  the  testing  voltage.  VM	 testing

voltage  is selected as 1.7V, 3.1V, 4.6V, 9.1V,

10.3V,	12.6V, 17.3V, 18.7V, 21.9V etc.

--  totally  9	settings. In testing, you simply

test the voltage status at pin	1.

<P>: To  test each pin whether it's voltage value is

same to that as showed	on the screen. VM pin is

assigned at pin 1, 5, 7, 9, 10, 11, 12, 13, 14,

15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26,

27, 28, 29, 30, 31, &32.

<ESC>: back to main menu

*** FUNCTION 3 : VH TESTING & ADJUSTMENT

To test and adjust VH voltage (VR2)

<S>: To  select  the  testing  voltage.  VH	 testing

voltage is selected as	3.0V, 4.5V, 6.0V, 18.6V,

11.9V,	14.2V, 18.9V, 20.3V, 23.7V

--  totally  9	 setting. In testing, you simply

test the voltage status at pin	21.

<P>: To  test  each	pin whether its	voltage	value is

same  to  that	 showed	on the screen. VH pin is

assigned  at pin 9, 11, 13, 19, 21, 23, 24, 25,

26, 27, 28, 29, 30 & 32.

<ESC>: back to main menu.

*** FUNCTION 4 : GROUND	TEST

To test ground.

To  test  whether pin 1, 11, 20, 23, 24, 25 28,

29  30	is from	0V to 0.3V or not , execute this

function.

<ESC>: back to main menu.

*** FUNCTION 5 : I/O TEST

To test I/O BUS.

'1'

'0'

<CR>: to exchange odd pins with even	pins

<ESC> : back to main menu

*** FUNCTION 6 : EXTRA TEST

To test the indicator light & the CLOCK.

�                                                       page 11-3

<1>: to  test  whether  BUSY LED is	normal or not. (

The light will	become flashing	if it is normal)

<2>: In executing this function, please test whether

pin 2,	3, 18 &19 have 2MHz frequency.

Press any key to quit.

<ESC>: back to main menu.

**** FUNCTION Q	: QUIT

Quit this testing.

########################################################