PURDUE UNIVERSITY

General Precision-4  (GP-4)

( PC Support Utilities System )

dated   : 22-Oct, 1995

Author  : Clyde Eckler

Company : Binghamton Simulator Company Inc.

On-line

Filename: \gp4\gp4.doc

TABLE OF CONTENTS

Section    Description                                               Page

-------    -----------                                               ----

1.0    INTRODUCTION  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

2.0    REQUIREMENTS  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

3.0    COMPONENTS .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

4.0    SOFTWARE FUNCTIONAL DECOMPOSITION  .  .  .  .  .  .  .  .

5.0    SOFTWARE TOOLS DESCRIPTION.  .  .  .  .  .  .  .  .  .  .

5.1    TAPE LOADER.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

5.2    GP UTILITY .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

5.3    GP LISTINGS.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

5.4    DDP  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

5.5    DAFG .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

5.6    DIAGNOSTICS.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

6.0    REFERENCES .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

7.0    DIRECTORY TREE.  .  .  .  .  .  .  .  .  .  .  .  .  .  .

1.0    INTRODUCTION

------------

This is a modification to do away with the need for the

GP4 teletwpe, tape unit, and card reader. These units

are replaced with an IBM PC connected to the GP4 through

the drum loader section. The tape unit control panel

operation will be the same with the PC. The teletype

and card reader operate on line, opposed to drum load on,

through the function called TTY in the PC.

The DAFG and DDP source cards were loaded into the PC

with a card reader. These cards are now files in the PC

that can be edited with a software editor instead of the

card punch. Likewise the title cards are now files in the

PC. Since the GP drum image and the GP source are the

same, the drum to tape function witch reads the drum to PC

will now be the GP cards

2.0    REQUIREMENTS

------------

120VAC 50-60 HZ

10 to 30 deg C operating temperature

3.0    COMPONENTS

----------

GP-4 computer

IBM PC with DOS 3.2 or higher, hard drive, floppy disk

Keithley PIO-12 pc board

Line printer

GP4 interface board BSC pn 1813-001

Interface cable     BSC pn 1816-001

See BSC prints 1812 through 1818

4.0   SOFTWARE FUNCTIONAL DECOMPOSITION

---------------------------------

The GP4 support utilities software are located in the \GP4 directory.

Under \GP4 there are 5 sub directorys.

1. \DRUM

2. \GP

3. \LIST

4. \DAFG

5. \DDP

6. \DIAG

1. \DRUM contians the 4 drum-tape loading functions.

T_DTA

T_TDA

T_DTO

T_TDO

After runing T_DTA, all the drums 64 quads will be

added to this directory. It is advised that a

floppy be made as a back up of the drum quads.

2. \GP contians G_MKS

G_A

G_D

G_F

G_S

G_VI.BAT

G_    LDK

3. \LIST contains L_MKL

L_LIST

L_EREF

L_AREF

L_BREF

and all the title cards for your simulator.

4. \DAFG contains F_COM

F_PRINT

F_LIST

F_VI

DTONE

TDONE

and the source cards for the DAFG quads. These files can be

edited the same as you would with the card punch.

The F_COM function can be used to compile

the source to a drum file. Then T_TDO function can be used

to load it to the GP4.

5. \DDP contains D_COM

D_DRUM

D_DDP

D_LIST

D_PRINT

D_COMP.BAT

D_VI

D_BP

D_DUAL

and the source cards for the DDP quads, with rec_bp.src for

the recievers and band patterning information. These files

can be edited the same as you would with the card punch. The

D_COM and D_DRUM functions can be used to

compile the source to a drum file. The T_DO function can be

used to load it to the GP4.

All GP4 drum image files are named as follows and are located under the

\GP4\DRUM directory.

1.1  1.2  1.3  1.4       Band #1  Quads 1->4

2.1  2.2  2.3  2.4       Band #2  Quads 1->4

3.1  3.2  3.3  3.4       Band #3  Quads 1->4

4.1  4.2  4.3  4.4       Band #4  Quads 1->4

5.1  5.2  5.3  5.4       Band #5  Quads 1->4

6.1  6.2  6.3  6.4       Band #6  Quads 1->4

7.1  7.2  7.3  7.4       Band #7  Quads 1->4

8.1  8.2  8.3  8.4       Band #8  Quads 1->4

9.1  9.2  9.3  9.4       Band #9  Quads 1->4

10.1 10.2 10.3 10.4       Band #10 Quads 1->4

11.1 11.2 11.3 11.4       Band #11 Quads 1->4

12.1 12.2 12.3 12.4       Band #12 Quads 1->4

13.1 13.2 13.3 13.4       Band #13 Quads 1->4

14.1 14.2 14.3 14.4       Band #14 Quads 1->4

15.1 15.2 15.3 15.4       Band #15 Quads 1->4

16.1 16.2 16.3 16.4       Band #16 Quads 1->4

All of the above files are text files that will look like GP instructions even

though some are DAFG and DDP files.

To read DDP or DAFG as octal drum image use program "D_PRINT" or "F_PRINT".

All GP source files that are created are named as follows and placed in the

\GP4\GP directory ;

011.src 012.src 013.src 014.src

021.src 022.src 023.src 024.src

031.src 032.src 033.src 034.src

041.src 042.src 043.src 044.src

051.src 052.src 053.src 054.src

061.src 062.src 063.src 064.src

071.src 072.src 073.src 074.src

081.src 082.src 083.src 084.src

091.src 092.src 093.src 094.src

131.src 132.src 133.src 134.src

141.src 142.src 143.src 144.src

All DAFG source files that are created are named as follows and placed in the

\GP4\DAFG directory ;

101.src 102.src 103.src 104.src

111.src 112.src 113.src 114.src

121.src 122.src 123.src 124.src

All DDP source files that are created are named as follows and placed in the

\GP4\DDP directory ;

151.src 152.src 153.src 154.src

161.src 162.src 163.src 164.src

154BP.SRC 164BP.SRC

All TITLE  files that are created are named as follows and placed in the

\GP4\LIST directory ;

EQUA.TIT

ARITHA.TIT

ARITHB.TIT

ARITHC.TIT

ARITHD.TIT

ARITHE.TIT

ARITHF.TIT

ARITHG.TIT

ARITHH.TIT

ARITHI.TIT

ARITHJ.TIT

ARITHK.TIT

ARITHL.TIT

ARITHM.TIT

ARITHN.TIT

ARITHO.TIT

ARITHP.TIT

BOOLA.TIT

BOOLB.TIT

BOOLC.TIT

BOOLD.TIT

BOOLE.TIT

BOOLF.TIT

BOOLG.TIT

BOOLH.TIT

BOOLI.TIT

BOOLJ.TIT

BOOLK.TIT

BOOLL.TIT

BOOLM.TIT

BOOLN.TIT

BOOLO.TIT

BOOLP.TIT

5.0   SOFTWARE TOOLS DESCRIPTION

--------------------------

The DOS version of the GP4 utilities can be entered by clicking on the dos

utilities or in dos, cd to gp4 and type gp4. A window will appear on the

screen with six choices. Use the up down arrows th choose one and hit cr.

5.1   TAPE LOADER

-----------

5.1.1

All tape modes execpt D/T ALL will go to a window and let you choose a band

and a quad with the arrow keys. After choosing a band-quad follow the

instructions on the screen. Allways start the gp4 load start before hiting

any key to start the PC. You can display the CMI/O DMA-A ADR on the gp4 to

watch the gp4 adr counter count. The PC will keep track of the load failures

by printing them out on the line printer in the tape all modes.

5.6.1 TTY

This function is used to emulate the origanal GP4

Teletype. It can be used to run the GP4 Online or

Offline utilities, and the DAFG and DDP compilers.

Allways use caps lock when in TTY, use small q to

return to DOS. When using the compilers to load cards

depress F1 for GP cards,

F2 for DAFG cards,

and F3 for DDP cards.

When loading DDP cards the function will automaticly

throw the send rec switch. To terminate a long

print out, depress the End key and type XXXX. This

will terminate the printout.

5.2   GP UTILITY

----------

5.2.1 EDIT GP QUAD

After selecting this program and before going to select a band-quad,

a window will appear with a selection of vi or edit. After selecting

the band-quad, the program will

now will make the xxx.src file for this quad.

Edit will now be called on this xxx.src file.

The user can now EDIT the quad. The user can delete lines

or add lines disregarding the drum address. When adding

lines, put anything in the drum address area, and anything

in the assembly area. Example to add a 22F252 after

drum address A001.

A000 12 A010 EQ#

A001 12 P000 EQ#

0 22 F252 0

A002 22 C000 LDK

A003 23 F400 STA

Save this using EDIT, then exit EDIT. Now the A is called.

It will evaluate this quad, and advise you that the checksum

for equation P000 should be changed. Go ahead and store this

quad when the program ask. Now recall GPEDIT and change the

checksum in EDIT. Now A will advise you that the quad is ok.

Notice also that the new instruction is now

A002 22 F252 LDK

and the spare will be one less.

5.2.2 DUMP

Dump will ask you for the equation or all.

Dump will print this equation on the line printer.

The xxx.src file has to be present for dump to work.

5.2.3 S

Use capital

when typing the letter of the core address.

This will surch for the core memory address specified

and print out on the line printer the drum address

and the instruction were this address is used in

this quad.The xxx.src file has to be present

for s to work.

5.2.4 F DUMP

F is similar to the f dump on the gp4 utility.

It will give the user the equation start instruction

with its drum adderss in octal and decimal.

The decimal drum location is usefull when useing vi,

sense vi works in decimal lines.

The xxx.src file has to be in

the working directory for f to work correctley.

5.2.5 LOAD CONSTANT

This function is used to give a double load constant

for a given decimal number. The user is advised to type

in the decimal number with a leading decimal point,

such as .1234567. The function will respond with

Load constants are

22A771

22F333

Or type -.1234567 the constants are

22I771

22F333

5.3   GP LISTING

----------

5.3.1 MAKE LIST

It will read the drum image

file and make a file called list_b.q. This file is used

by list gp to output the listing to a line printer.

If make list finds a core that dose not have a title,

it will advise the user on the line ptr of the missing title.

At the conclusion of the pgm it will ask the user if the quad is ok,

meaning there are no missing titles. If ok make list will make 3

files a_refb.q , b_refb.q, and e_refb.q ,to store the referance

data. These files are used for referance core lists. Make list

could take up to 5 min to run a full quad, the disk light

will be blinking a lot during the running of make list.

If any changes are made in one quad, make list will have to

be run to update the listing and core referance.

Before running make list the following files should be in the

directory.

All the core title files arithx.tit boolx.tit and equa.tit.

The x in arith and bool stands for the letter of the core.

There should be 16 arith and 16 bool title files.

5.3.2 LIST GP

Program will ask the user for equation nbr for printing

one equation, or all for the whole quad. Listing will

output to the line printer. Make sure a file called cust

is in the directory. Cust can be made by vi and is used

in the listing as the top line of aech page of listing.

Example would be"PURDUE 727-1".

5.3.3 ARITH REF

Program will ask user for the arithmetic core address.

The referance will be output on the line printer.

5.3.4 BOOL REF

Program will ask user for the boolean core address.

The referance will be output on the line printer.

5.3.5 EQUA REF

Program will ask user for the equation number.

The referance will be output on the line printer.

5.4   DDP

---

5.4.1 DDP COM

Bands 15 and 16 src files should be present

in the directory GP4\DDP. These src files were

created by the original ddp card decks. The

user can use vi to modify these files. The

eight card layout is the same as the original

gp4 ddp, including the column dependent items.

Ddpcom will create two files for each ddp quad,

xxx.dat and xxx.lim. Xxx=the band-quad. Xxx.dat

are the 8 data words for each station, and

xxx.lim is the 6 limit words for each station.

From these files a drum image file will be

created and put into the drum dir. The user

can now go to the tape loader-t/done to

load this new quad on the gp4 drum.

If any errors are encountered, DDPCOM will advise

the user of the error and will not compile this

quad until the error has been corrected in the

.src file.

5.4.1.a DDP COM DUAL

If the simulator is set up to rum dual ddp, the

D_COMP.BAT has to be changed to reflect the

dual option. When compiling 15.1, D_DUAL will

insert the 8 data words in 15.1 drum image,

and insert the 6 limit words in 16.1. The

process will be reversed when compilling 16.1.

5.4.2 DDP

The program will prompt the user to type the station

number a space and s,p,or x. Where s = print on screen,

p = print on line printer, and x to quit.

After the station nbr is typed the station's eight

data words and six limit words are printed.

The user can also print the spare station numbers left

in the ddp quad by typing 0 space s or p, for

screen or line printer.

5.4.3 EDIT DDP

This allows the user to edit the ddp sourc using vi or edit.

The quad labeled P is for the band patterning, reciever address

sourc files.

5.4.4 LIST DDP

This will list on the line printer the first card only of each

station in this quad.

5.4.5 PRINT DRUM

It will ask the user for the start address and

last address to be printed on the line printer.

This program is usefull for printing out the octal

drum locations for the ddp drum data.

The data will be printed out on the line printer

along with the drum address.

5.4.6 BAND PATTERN

This function is called under the DAFG menu,

because there was not any room under the DDP.It will

only take bands 15 and 16 quad 4. The program

is used for changing the GP4 band patterning

or the DDP reciever addresses. This data is in

a file called 154BP.SRC, or 164BP.SRC. The program will take

the data in 154bp.src and insert it in the ddp

band quad 4. TDONE can be used then to load this

to the GP4.

5.4.7 154BP_SRC or 164BP.SRC

This is not a program but a file that the

user has to create. It contains the reciever

addresses and the band patterning infomation

for the fourth ddp quad. The file has to be

made up in a particular way.

<R will start the receiver addresses

<G will start the gp band pattern

<D will start the dafg band pattern

# preceds the pattern number

$ will start each line in R ,G and D.

Following is an example

<GP BAND PAT #1

$ 01 03 04

$ 01 02 05

$ 01 06 07

$ 01 02 08

$ 01 03 04

$ 01 02 05

$ 01 06 07

$ 01 02 09

$ 00 00 00

<GP BAND PAT #2

$ 14 00 00

<GP BAND PAT #3

$ 13 00 00

<DAFG BAND PAT #1

$ 10 11 12

$ 00 00 00

<DAFG BAND PAT #2

$ 10 00 00

<DAFG BAND PAT #3

$ 12 00 00

<REC ADR FREQ LONG LAT  DATA

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1 STANDARD-2

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1 APPLS TO FOLLOWING

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2 EAL DC-9/DC-8 FINAL

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2 CAL DC-9/720 FINAL

$ C000 C101 C201 B000 MIDDLE MARKER         CKPT 1 ALI DC-9/DC-8

$ C006 C101 C201 B060 VHF NAV-1             CKPT 1 SAS DC-9/DC-8

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1 UAL 727/DC-8

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1 AAL 727 FINAL

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2 PAA 707 DUAL-1

$ C045 C141 C241 B450 LOW FREQ ADF -2       CKPT 2 PAA 707 DUAL-2

$ C040 C141 C241 B400 MIDDLE MARKER         CKPT 2 PAA 727 DUAL

$ C046 C141 C241 B460 VHF NAV-1             CKPT 2 SBA 727

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1 NAL 727

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1 NWA 727/707

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2 ADM DC-9/DC-8

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2

$ C001 C101 C201 B010 OUTER MARKER          CKPT 1

$ C007 C101 C201 B070 VHF NAV-2             CKPT 1

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2

$ C041 C141 C241 B410 OUTER MARKER          CKPT 2

$ C047 C141 C241 B470 VHF NAV-2             CKPT 2

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2

$ C002 C101 C201 B020 AIRWAYS MARKER        CKPT 1

$ C010 C101 C201 B100 DME-1                 CKPT 1

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2

$ C042 C141 C241 B420 AIRWAYS MARKER        CKPT 2

$ C050 C141 C241 B500 DME-1                 CKPT 2

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2

$ C003 C103 C203 B030 GCA-1                 CKPT 1

$ C011 C101 C201 B110 DME-2                 CKPT 1

$ C004 C101 C201 B040 LOW FREQ ADF-1        CKPT 1

$ C005 C101 C201 B050 LOW FREQ ADF-2        CKPT 1

$ C044 C141 C241 B440 LOW FREQ ADF-1        CKPT 2

$ C045 C141 C241 B450 LOW FREQ ADF-2        CKPT 2

$ C043 C143 C243 B430 GCA-2                 CKPT 2

$ C051 C100 C241 B510 DME-2                 CKPT 2

5.5   DAFG

----

5.5.1 DAFG COM

Bands 10, 11, and 12 src files should be present

in the directory GP4\DAFG. These src files were

created by the original dafg card decks. The

user can use vi to modify these files. The

card layout is the same as the original

gp4 dafg, including the column dependent items.

Dafgcom takes the xxx.src files and creates

gp4 drum image files that can be loaded on the

drum using tdone.

If any errors are encountered, DAFGCOM will advise

the user of the error and will not compile this

quad until the error has been corrected in the

.src file.

5.5.2 EDIT DAFG

This allows the user to edit the dafg sourc using vi or edit.

5.5.3 LIST DAFG

This will list the dafg quad the same way that the gp4 dafg resume.

5.5.4 PRINT DRUM

It will ask the user for the start address and

last address to be printed on the line printer.

This program is usefull for printing out the octal

drum locations for the dafg drum data.

The data will be printed out on the line printer

along with the drum address.

5.6    DIAGNOSTICS

---------

5.6.1 DIAG

Diag dir contians a drum load of the gp4 diagnostics.

The following is what is on each band.

BAND

1   GP diag

2   DAFG and DDP diag

3   DAFG comp

4   DDP comp

5

6   DDP comp

7   DAFG comp

8.1 Sine d/a test

9

10.4 DAFG diag

11.4 DAFG diag

12.4 DAFG diag

13

14   utility

15   DDP diag

16   BP

5.6.2 TAPE MODES

The use of these tape loaders will have to be run under DOS.

The path is C:\GP4\DIAG.

In this dir are these executebles, I_TDO and I_TDA.

To use these programs type in the name of the program

followed by the band.quad ,example I_TDO 1.1 will load

the first quad of the gp diag on the drum.

6.0    REFERENCES

----------

LP-5536-1      GP-4 computer operation and maintenance manual

LP-5535-4      GP-4 computer programming manual

LP-5520-A      GP-4 computer programmer's reference manual

ER-835         GP-4 on-line utility package operator's manual

ER-793         GP-4 utility program operator's manual (Off-line)

ER-889/vol.III GP-4 DAFG utility program operator's manual

ER-973/vol.IV  GP-4 DDP compiler user operator's manual

ER-791         GP-4 DDP programming manual

LR- 672        GP-4 DDP dual compiler

7.0    DIRECTORY TREE

--------------

The GP4 layout is reflected below. The Dirrectory names have a \ in front

of them. All names without dots are exe files. To run the GP4 programs,

change dir to \GP4, and type GP4. The programs are window driven after this.

\C:

|

\GP4

|

GP4.DOC

EGAVGA.BGI

TRIP.CHR

GP4.BAT

ECEC

________________________________|______________________________________

|             |             |             |             |             |

\DRUM         \GP           \LIST         \DDP          \DAFG         \DIAG

|             |             |             |             |             |

T_TDO         G_VI.BAT      L_MKL         D_COMP.BAT    F_COM         I_TDO

T_DTO         G_MKS         L_LIST        D_COM         F_VI          I_TDA

T_TDA         G_A           L_AREF        D_DRUM        F_LIST

T_DTA         G_D           L_BREF        D_DDP         F_PRINT

T_TTY         G_S           L_EREF        D_VI

G_F                         D_LIST

G_LDK                       D_PRINT

D_BP

D_DUAL

1.1           011.src       arith_.tit    151.src       101.src       1.1

|               |          bool_.tit        |             |           |

16.4          134.src       equa.tit      164.src       124.src       16.4

cust

list_1.1      151.lim

|            |

list_13.4     164.lim

aref_1.1

|         151.dat

aref_13.4        |

bref_1.1      164.dat

|

bref_13.1     154bp.src

eref_1.1      164bp.src

|

eref_13.4