--------------------------------------------------------------

PSEUDCRT.MAC	7/11/82		Eric Forbes

--------------------------------------------------------------

PSEUDCRT.MAC is a modified BIOS CONSOLE OUT routine to allow

a simple TV-TYPEWRITER to be used with software written  for

the more sophisticated addressable cursor terminals.

The values used as shown in the EQU's are for a VT-52.

Calls to CONOUT are checked to see if it is an ESCAPE sequence

and if it is the cursor is moved using the control  characters

available to the simple tv-typewriter.

The control characters required are:-

Back space		normally hex 08

Horizontal tab		normally hex 09

Line feed		normally hex 0A

Carriage return		normally hex 0D

Reverse line feed	normally hex 0E

Erase to end of line	normally hex 0F

Being able to switch the cursor off while moving it makes for

a 'cleaner' screen, but is not necessary.

I have this in my  BIOS  and  a  RE-6416  CRT  controller, the

combination works  very  well.  It will do til I get around to

building a better controller.

Happy computing,

Eric Forbes.

##############