Date: 21 July 1981 2201-EDT (Tuesday)

From: Bill Sholar

To:   All

Re:   TRS-80 Model II MODEM Equates & Ports

Since the documentation on the Mod 2 TRS-80 does not bother

to discuss hardware, the following port addresses and equates for

MODEM206.ASM programs might prove useful to anyone using this machine:

MODCTLP	EQU	0F6H	;status/control port A

MODSNDB	EQU	04H	;bit 2 indicates ready to send

MODSNDR	EQU	04H	;  with the bit set to 1

MODRCVB	EQU	01H	;bit 0 indicates character received

MODRCVR	EQU	01H	;  with the bit set to 1

MODDATP	EQU	0F4H	;data port A

These values, plugged into MODEM.ASM, allow the TRS-80 Mod 2 to

function without any special system calls or other variations from the

existing MODEM206.ASM program, assuming the port has been set up using the

SETUP (Pickles & Trout's CPM), CONFIG (Lifeboat's CPM) or SET (Cybernetics'

CPM) utility for the appropriate baud rate, etc.

##################################################################################################