OSTRICKS.DOC  OSBORNE 1 UNDOCUMENTED TRICKS

taken from 6/82 Microcomputing letter p. 24.

BAUD RATE:

The rs-232c baud rate can be set using the

following:

B03	=	056H	;300  baud

B12	=	057H	;1200 baud

B96	=	054H	;9600 baud

BASE	=	0E500H	;BIOS JUMP TABLE ON

; MODIFIED UNITS, ELSE

;0E500H ON UNMODIFIED.

MVI	C,B96	;..for 9600

CALL	BASE+03CH   ;set the new rate

Possibly these rates can also be doubled by

changing the jumper in the OS-1 as when the

book explains how to get 600 and 2400 baud.

BOOTING RIGHT-SIDE DISK DRIVE:

On reset, when you get the prompt for for

a carriage return to boot normally, reply with

a double-quote mark  "

The right side drive is then A:, and the

left side drive is B:.

This is useful if your left side drive is

temporarily not operating properly (or all all).

PRINTER TOGGLE:

CP/M Control-P is the normal toggle. If addr

0DA0DH is non-zero, the printer will receive an

echo of the console output.

PROGRAM FUNCTION KEYS:

Set them up as in the manual. They are then

invoked with a command 0-9, which is not clear

in the manual.

WORDSTAR BACKSPACE:

After the modification is applied to your

computer, the back-arrow key (which used to be

the delete char key), is now the cursor-left key.

Delete is Control - (minus) (undocumented).

MODIFICATION GOOD NEWS:

1. The capital lock now works for all char.

2. All programs can handle auto-scroll.

MODIFICATION BAD NEWS:

1. In Wordstar, when you auto-scroll, the

display does an annoying left/right dance when

doing a rapid vertical scroll.

2. The upgrade BIOS uses registers X and Y.

CP/M was written in 8080 code where these do

not exist. CP/M programs which use X and Y, and

worked fine previously, may have to be altered

to save X and Y on the stack prior to CP/M calls.

---------------------------------------------

#may have to be altered

to save X and Y on the stack prior to CP/M calls.

--------------------------------