***********************************

*           ntxt3 notes:          *

*      including source code      *

***********************************

This program owes its existence to the original Z80

version,  TXT.COM,  by Simon Ewins (416) 484-9663

(e-mx rcp/m) and to the 8080 adaptation, N-TXT.COM,

by Harry Kaemmerer, CP/M-NET EAST (201) 249-0691,

whose fuller explanation may be found below.

PAGING added (and display routine changed)

by Walt Wheeler (YOU)

and Justus Addiss (Sigma Xi),

12 June 84

========================================================================

=========================================================================

NTXT3.COM provides a means for creating a RUNNABLE docu-

ment...that, for making a document accessable to users

unfamiliar with CP/M, or, in another example provided by

Simon Ewins,

"We have used it to help on my rcp/m

with the problem of some users typing QUIT

or LOGOFF etc. instead of BYE.

"Instead of having to create an assembler file

with DB lines and write a routine to print a

message that says "Please use BYE to log off

this system......" (plus other info), and then

assemble it and then load it, we simply follow

the procedure outlined (below) and create an

'instant' .com file with the editing capabil-

ities of ws.

"We have also used it to display help files in

a business environment. The operator simply types

the name of the item she/he wishes to know more

about and gets an instant listing."

The following instructions refer to use with WordStar, but could

equally well be implemented with any editor (including, ugh, ED)

capable of creating plain vanilla ASCII files.

1.  Open a file using the NON-document mode, under the

name which you want to use when the TEXT file is displayed... for

example, THISSYSTEM.COM.  (No, Virginia, WS will NOT balk at editing

a file of type ".COM")

2.  Use ^KR to read in the file NTXT3.COM as the initial

part of the new runnable file. (The file NTXT3.COM amounts to a

machine language header which includes all the code necessary to

display and page the test you'll be adding.)

3.  Locate the "?" and NUL ("^@") which delimit the begin-

ning and end of the text buffer in the "copy" you've just read in.

(They're side by each, as distributed.)

======> NOTE: the "^@" in NTXT3.COM really is a NUL <======

======> and NOT -- again, NOT -- a CARET/AT_SIGN   <======

4.  Now, use ^KR to read in a plain ASCII (but NOT a WS

document-mode)file which has been created previously, or use WS's

capabilities to insert text, graphics, or whatever,  between the

delimiters.  (The closing NUL is included in the file because some

systems in common use -- including the Osborne and Otrona "Attache"

have no direct method of generating a NUL.)

5.  If you wish to include notes which will NOT be displayed

when the file is run, add them after the final NUL.

6.  WARNING:  If you somehow omit a closing NUL, undefined

and sometimes unpleasant results will occur.

=========================================================

To use the paging function, insert a TILDE (07EH):

EITHER immediately before the first char-

acter which you want to have appear on the next

screenful (after the <CR><LF> pair at the end of

any given page)

OR  (together with a <CR><LF> pair as

terminators) on an otherwise blank line between

the two pages of text in question.

NOTE:	Putting the TILDE at the end of the final

line of a page, before the <CR><LF> will cause the

"Hit any key...." prompt to display WITHOUT any

spaces or other breaks, immediately after the last

character of your screenful.

If you follow the above suggestions a screenful (page)

should not exceed 23 lines, as the "Hit any key..."

does force a newline and thus will force the first line

of a 24 line page to scroll off screen, very possibly

before the user has a chance to read it.

====================================================

KNOWN DEFICIENCIES, LIMITATTIONS:

Because the paging routine uses BDOS Function 6 (for

good and sufficient reason (determining that reason is

left as an exercise for the reader), and because our

ambition gave out before implementing the usual

methods of aborting a program or halting the display,

^C, ^S (and sometimes ^Z) DO NOT WORK.  As the

British have it, "in for a penny, in for a pound."

Should someone add the necessary code to do that,

we'd be pleased ...and pleased to see same.  Bug

reports, too, please.			wcw,ja

======================================================

SOURCE CODE:

;--------------------------------------------------------------------

;	NTXT3.ASM  adapted from:

;

; The original z80 TXT.COM version

; 	by Simon Ewins

; 	(416) 484-9663 (e-mx rcp/m)

;

; The 8080 source for the first two lines or so of:

; 	N-TXT.COM message utility January 1, 1984

; 	by Harry Kaemmerer

; 	CP/M-NET EAST (201) 249-0691

;

; added PAGING, allowing the creator of a file to implement

; paging, by inserting a tilde ("~") in the source text file,

; to halt CONOUT until the reader strikes a key.  Suggest including

; a tilde not less often than every 22nd line.

;	CF:  the main display functions used here, with those

;            of the earlier versions.  No claim is made for any

;            improvement - these simply happen to be different.

;

;    Walt Wheeler, Yankee Osborne Users (Y.O.U.)  06.12.84

;	and Justus Addiss, Sigma Xi

;---------------------------------------------------------------------

;

BDOS    EQU   	5       	;system call entry point

CONOUT  EQU   	2       	;print string to console

DCIO	EQU	6		;Direct Console I/O, no echo to screen

CR      EQU	0DH       	;carriage return

LF      EQU	0AH       	;line feed

TILDE	EQU	07EH		;tilde

;

;--------------------------------------------------------------------

; ----  START OF PGM  ----

;

ORG	100H		;

;

START:	LXI	H,0		;

DAD	SP	 	;get old stack pointer

SHLD	OLSTACK	 	;lets save it for later

LXI	SP,STACK 	;set up a new stack pointer

;

LXI	H,BUFFER	;point to text

;

CALL	COMSG

;

EXIT:	LHLD	OLSTACK		;get the old stack pointer back

SPHL			;restore it, as it has the pgm rtn adrs

RET			;...and go to the program return address

;

;get msg pointed to by HL, test for NUL or TILDE

COMSG:	MOV	A,M		;get a char

ORA	A		;test for match w/Nul (zero)

RZ			;and return if it is

CPI	TILDE		;else test for tilde (marks page break)

CZ	PGBRK		;if its a pgbrk marker, go to that routine

CALL	CO		;else spit it out to console using CO

INX 	H		;point to next char...

JMP	COMSG		;and loop

;

;IF NOT NUL OR TILDE, send char in A to CON: (screen)

CO:	PUSH 	B		;\

PUSH 	D		; save registers

PUSH 	H		;/

MVI	C,CONOUT	;put read funct in C register

MOV	E,A		;move the char in A to E

CALL	BDOS		;send to screen (CON: device)

POP 	H		;\

POP 	D		; restore registers

POP 	B		;/

RET			;and go back to calling routine

;

;IF char in A is TILDE, halt output to CON: until

;user strikes a key. Note that ^C,^S will NOT interrupt

PGBRK:	PUSH B			;save the registers

PUSH D			;in what is partly an excess

PUSH H			;of caution.

LXI	H,WMSG		;put address of page break msg in HL

CALL	COMSG		;spit the "hit any key" msg out to CON:

WAIT:	MVI	E,0FFH		;set the flag for DCIO, which halts till

MVI	C,DCIO		;it gets a char from CON:, setup for BDOS 6

CALL	BDOS		;do it

ORA	A		;test for match w/00

JZ	WAIT		;loop

MVI	A,0H		;lose the char typed by user

POP H			;restore the registers

POP D			;

POP B			;

RET			;go back to calling routine

;

;wait message

WMSG:	DB	'Hit any key to continue',CR,LF,0

;

;--------------------------------------------------------------------

;   STACK AREA

;

;a portion of OLSTACK will be overwritten when

;this program executes.  That is of no consqeunce,

;since the "Insert your text..." message is NOT

;needed at runtime. This incidently saves some TPA.

;

OLSTACK	DB	'-->> Insert your Text between'	;save old stack here

DB	' the "?" and the "^@"',CR,LF	;and make room for the

STACK	EQU	$            			;new stack, starting here

DB	'?'				;start of buffer marker

BUFFER	EQU	$				;buffer used by wordstar

;--------------------------------------------------------------------

; Message space starts here, and is as large as your TPA -512 bytes

;--------------------------------------------------------------------

DB	0,26		;force a NULL and an END of file marker

END			;to keep the WordStar file clean

=============================================================================

End of Document

#############################################################